IBM Connectors Pinout 1/4

Monochrome Display Adapter (MDA) Connector

��������������������������������������������������

Pin  Description        Direction

----------------------------------

1    Ground             -

2    Ground             -

3    Not used           -

4    Not used           -

5    Not used           -

6    +Intensity         Out

7    +Video             Out

8    +Horizontal        Out

9    -Vertical          Out

Color Graphics Adapter (CGA) Connector

����������������������������������������

Pin  Description        Direction     Notes

----------------------------------------------

1    Ground             -             Composite Jack

2    Ground             -

3    Red                Out           Center=Comp Video Signal �1.5 Volts

4    Green              Out           Outside=Ground

5    Blue               Out

6    +Intensity         Out

7    Reserved           -

8    +Horizontal Drive  Out

9    -Vertical Drive    Out

Enhaced Graphics Adapter (EGA) Connector

������������������������������������������

Pin  Description                  Direction

-------------------------------------------

1    Ground                       -

2    Secondary Red                Out

3    Red                          Out

4    Green                        Out

5    Blue                         Out

6    Secondary Green/Intensity    Out

7    Secondary Blue/Mono video    Out

8    Horizontal Retrace           Out

9    Vertical Retrace             Out

Video Graphics Array (VGA) Connector

���������������������������������������

Pin  Description                  Direction

-------------------------------------------

1    Red                          Out

2    Green                        Out

3    Blue                         Out

4    Monitor ID 2                 In

5    Digital Ground               -

6    Red Analog Ground            -

7    Green Analog Ground          -

8    Blue Analog Ground           -

9    Key (Plugged Hole)           -

10   Sync Ground                  -

11   Monitor ID 0                 In

12   Monitor ID 1                 In

13   Horizontal Sync              Out

14   Vertical Sync                Out

15   Reserved                     -

Pin Assignment for the Game Adapter

������������������������������������

Pin  Description

-----------------

1    +5 VDC

2    Button 1

3    Position 0  (X Coordinate)

4    Ground

5    Ground

6    Position 1  (Y Coordinate)

7    Button 2

8    +5 VDC                  JOYSTICK "A"

----------------------------------------

9    +5 VDC                  JOYSTICK "B"

10   Button 3

11   Position 2  (X Coordinate)

12   Ground

13   Position 3  (Y Coordinate)

14   Button 4

15   +5 VDC

Parallel Port Connector

�������������������������

Pin  Description                                                 Direction

---------------------------------------------------------------------------

1   ~Strobe (Normal=High, Data read-in when Low)                 Out

2   +Data Bit 0                                                  Out

3   +Data Bit 1                                                  Out

4   +Data Bit 2                                                  Out

5   +Data Bit 3                                                  Out

6   +Data Bit 4                                                  Out

7   +Data Bit 5                                                  Out

8   +Data Bit 6                                                  Out

9   +Data Bit 7                                                  Out

10  ~Acknowledge (�5us pulse,low=data rcvd and printer is ready) In

11  +Busy                                                        In

12  +Paper End (High=printer out of paper)                       In

13  +Select (printer is in the selected state)                   In

14  ~Autor Feed (low=paper automat fed one line after printing)  Out

15  ~Error                                                       In

16  ~Initializae Printer                                         Out

17  ~Select Input                                                Out

18  -Data Bit 0 Return (Ground)                                  In

19  -Data Bit 1 Return (Ground)                                  In

20  -Data Bit 2 Return (Ground)                                  In

21  -Data Bit 3 Return (Ground)                                  In

22  -Data Bit 4 Return (Ground)                                  In

23  -Data Bit 5 Return (Ground)                                  In

24  -Data Bit 6 Return (Ground)                                  In

25  -Data Bit 7 Return (Ground)                                  In

IBM Connectors Pinout 2/4

Pin Assignment for the Serial Port (RS-232C)

��������������������������������������������������

Pin                  Description                            Signal   Direction

------------------------------------------------------------------------------

PC <--> AT

25-pin <--> 9-pin

------------------

1                 Chassis Ground                         -         -

2   <-->    3     Transmit Data                          TD        Out

3   <-->    2     Receive Data                           RD        In

4   <-->    7     Request To Send                        RTS       Out

5   <-->    8     Clear To Send                          CTS       In

6   <-->    6     Data Set Ready                         DSR       In

7   <-->    5     Signal Ground                          SG        -

8   <-->    1     Carrier Detect                         CD        In

9                 +Transmit Current Loop Return (20ma)   -         Out

11                 -Transmit Current Loop Data (20ma)     -         Out

8                 +Receive Current Loop Data (20ma)      -         In

20   <-->    4     Data Terminal Ready                    DTR       Out

22   <-->    9     Ring Indicator                         RI        In

25                 -Receive Current Loop Return (20ma)    -         In

Keyboard Connector Signalas

�����������������������������

Pin name           5 pin DIN         6 pin mini-DIN       6 pin SDL

--------------------------------------------------------------------

Keyboard data         2                    1                  B

Ground                4                    3                  C

+5V                   5                    4                  E

Keyboard clock        1                    5                  D

Not connected         -                    2                  A

Not connected         -                    6                  F

Not connected         3                    -                  -

Plug                 Socket

1 *   * 3             3 o   o 1

5 pin DIN             4 *     * 5           5 o     o 4

*                     o

2                     2

Plug                 Socket

5 * � * 6             6 o � o 5

6 pin mini- DIN       3 *     * 4           4 o     o 3

* *                   o o

1 2                   2 1

Plug                  Socket

���������������Ŀ      ���������������Ŀ

� ������������� �      � ������������� �

6 pin SLD          � � � � � � � ��      � � � � � � � ��

� A B C D E F �        � F E D C B A �

���������������        ���������������

5 pin DIN are used on most computers today. 6 pin mini DIN are used on the

IMB PS/2 series. 6 pin SDL (Shielded Data Link) is a type of shielded

connector created by AMP and used by IBM and others for keyboards cables.

PC, XT and AT Power-Supply Connections

����������������������������������������

Connector       AT                      PC, XT

---------------------------------------------------------

P8-1            Power Good (+5)         Power Good (+5)

P8-2            +5                      Key (No connect)

P8-3            +12                     +12

P8-4            -12                     -12

P8-5            Ground                  Ground

P8-6            Ground                  Ground

P9-1            Ground                  Ground

P9-2            Ground                  Ground

P9-3            -5                      -5

P9-4            +5                      +5

P9-5            +5                      +5

P9-6            +5                      +5

P10-1           +12                     +12

P10-2           Ground                  Ground

P10-3           Ground                  Ground

P10-4           +5                      +5

P11-1           +12                     +12

P11-2           Ground                  Ground

P11-3           Ground                  Ground

P11-4           +5                      +5

P12-1           +12                     -

P12-2           Ground                  -

P12-3           Ground                  -

P12-4           +5                      -

IBM Connectors Pinout 4/4

Enhanced Small Device Interface (ESDI):

----------------------------------------

34-pin Control Connector:               20-pin Data Connector:

Signal    Pin number   Signal               Signal   Pin number    Signal

���������Ŀ                                 ���������Ŀ

Ground   ��1     2Ĵ   ~HD SLCT 2(3)     ~DRV SLCT   ��1     2Ĵ   ~SECTOR

Ground   ��3     4Ĵ   ~HD SLCT 2(2)    ~CMD COMPL   ��3     4Ĵ   ~ADDR MK EN

Ground   ��5     6Ĵ   ~WRITE GATE          Ground   ��5     6Ĵ   Ground

Ground   ��7     8Ĵ   ~CNFG/STATUS     +WRITE CLK   ��7     8Ĵ   -WRITE CLK

Ground   ��9    10Ĵ   ~XFER ACK            Ground   ��9    10Ĵ   +RD/REF CLK

Ground   ��11   12Ĵ   ~ATTENTION      -RD/REF CLK   ��11   12Ĵ   Ground

Ground   ��13   14Ĵ   ~HD SLCT 2(0)    +NRZ WRITE   ��13   14Ĵ   -NRZ WRITE

Key   ��15   16Ĵ   ~SECTOR              Ground   ��15   16Ĵ   Ground

Ground   ��17   18Ĵ   ~HD SLCT 2(1)     +NRZ READ   ��17   18Ĵ   -NRZ READ

Ground   ��19   20Ĵ   ~INDEX               Ground   ��19   20Ĵ   ~INDEX

Ground   ��21   22Ĵ   ~READY                        �����������

Ground   ��23   24Ĵ   ~XFER REQ

Ground   ��25   26Ĵ   ~DRV SLCT 1

Ground   ��27   28Ĵ   ~DRV SLCT 2

Ground   ��29   30Ĵ   Reserved

Ground   ��31   32Ĵ   ~READ GATE

Ground   ��33   34Ĵ   ~CMD DATA

�����������

Note: ~ means active low.

Integrate Device Electronics (IDE) or AT BUS pinout:

------------------------------------------------------

AT BUS 40-pin Connector:

Signal   Pin number    Signal

���������Ŀ

~RESET   ��1     2Ĵ   Ground

D7   ��3     4Ĵ   D8

D6   ��5     6Ĵ   D9

D5   ��7     8Ĵ   D10

D4   ��9    10Ĵ   D11

D3   ��11   12Ĵ   D12

D2   ��13   14Ĵ   D13

D1   ��15   16Ĵ   D14

D0   ��17   18Ĵ   D15

Ground   ��19   20Ĵ   Key

Reserved   ��21   22Ĵ   Ground

~IOW   ��23   24Ĵ   Ground

~IOR   ��25   26Ĵ   Ground

Reserved   ��27   28Ĵ   ALE

Reserved   ��29   30Ĵ   Ground

IRQ 14   ��31   32Ĵ   ~I/O CS16

A1   ��33   34Ĵ   ~PDIAG

A0   ��35   36Ĵ   A2

~CS1FX   ��37   38Ĵ   ~CS3FX

~SLAVE PRESEENT   ��39   40Ĵ   Ground

�����������

Note: ~ means active low.

Small Computer System Interface (SCSI):

----------------------------------------

Unshielded Internal                Shielded "D" Style

Header Connector:                  External Connector:

Signal    Pin number   Signal               Signal   Pin number    Signal

���������Ŀ                                 ���������Ŀ

Ground   ��1     2Ĵ   DB 0                 Ground   ��1    26Ĵ   DB 0

Ground   ��3     4Ĵ   DB 1                 Ground   ��2    27Ĵ   DB 1

Ground   ��5     6Ĵ   DB 2                 Ground   ��3    28Ĵ   DB 2

Ground   ��7     8Ĵ   DB 3                 Ground   ��4    29Ĵ   DB 3

Ground   ��9    10Ĵ   DB 4                 Ground   ��5    30Ĵ   DB 4

Ground   ��11   12Ĵ   DB 5                 Ground   ��6    31Ĵ   DB 5

Ground   ��13   14Ĵ   DB 6                 Ground   ��7    32Ĵ   DB 6

Ground   ��15   16Ĵ   DB 7                 Ground   ��8    33Ĵ   DB 7

Ground   ��17   18Ĵ   DB P                 Ground   ��9    34Ĵ   DB P

Ground   ��19   20Ĵ   Ground               Ground   ��10   35Ĵ   Ground

Ground   ��21   22Ĵ   Ground               Ground   ��11   36Ĵ   Ground

Reserved   ��23   24Ĵ   Reserved           Reserved   ��12   37Ĵ   Reserved

Open   ��25   26Ĵ   TERMPWR                Open   ��13   38Ĵ   TERMPWR

Reserved   ��27   28Ĵ   Reserved           Reserved   ��14   39Ĵ   Reserved

Ground   ��29   30Ĵ   Ground               Ground   ��15   40Ĵ   Ground

Ground   ��31   32Ĵ   ~ATN                 Ground   ��16   41Ĵ   ~ATN

Ground   ��33   34Ĵ   Ground               Ground   ��17   42Ĵ   Ground

Ground   ��35   36Ĵ   ~BSY                 Ground   ��18   43Ĵ   ~BSY

Ground   ��37   38Ĵ   ~ACK                 Ground   ��19   44Ĵ   ~ACK

Ground   ��39   40Ĵ   ~RST                 Ground   ��20   45Ĵ   ~RST

Ground   ��41   42Ĵ   ~MSG                 Ground   ��21   46Ĵ   ~MSG

Ground   ��43   44Ĵ   ~SEL                 Ground   ��22   47Ĵ   ~SEL

Ground   ��45   46Ĵ   ~C/D                 Ground   ��23   48Ĵ   ~C/D

Ground   ��47   48Ĵ   ~REQ                 Ground   ��24   49Ĵ   ~REQ

Ground   ��49   50Ĵ   ~I/O                 Ground   ��25   50Ĵ   ~I/O

�����������                                 �����������

Note: ~ means active low.

Very 73 from

������������������������������������ͻ

� Michel, EA1DOF @ EA1PH.EAVA.ESP.EU �

������������������������������������ͼ