AO 6.5 ANTENNA OPTIMIZER

Copyright 1995 by Brian Beezley, K6STI

All Rights Reserved

AO 6.5 Antenna Optimizer.......................1

Running AO.....................................1

Antenna Files..................................2

Coordinate System..............................3

Wires, Segments, and Pulses....................3

Automatic Wire Segmentation....................5

Automatic Segment-Length Tapering..............5

Pulse Identification...........................6

Sources........................................7

Loads..........................................8

Free-Space Symmetric Antennas.................10

Ground........................................11

Wire-Conductivity Losses......................14

Standing-Wave Ratio...........................15

Symbolic Dimensions...........................15

Symbolic Expressions..........................17

Shift and Rotate..............................17

Polarization Components.......................19

Near Field....................................20

Far Field.....................................21

3-D Geometry/Current Display..................21

3-D Pattern Display...........................22

2-D Pattern Generation........................23

2-D Pattern Plots.............................23

Screen Colors and Default Menu Settings.......26

Printing The Screen...........................26

Graphics Image Files..........................27

Aborting Calculations.........................27

Automatic Frequency Sweep.....................27

Batch Mode....................................28

Algorithm Limitations and Corrections.........29

Memory........................................31

SET Commands..................................33

Additional Information........................39

Index.........................................40

---- AO 6.5 ANTENNA OPTIMIZER ----------------------------------

AO is an enhanced version of the MININEC antenna-

analysis program combined with an automatic optimizer.  AO is

more powerful than MININEC, more accurate, easier to use,

faster, and has many additional features.  AO can analyze and

optimize almost any antenna made of wire or tubing.  However,

the program is not suitable for designs employing dielectrics,

ferrites, or surfaces.

AO is based on the MININEC 3.13 antenna-modeling

software developed by J.C. Logan and J.W. Rockway at the U.S.

Naval Ocean Systems Center.  AO uses a modified algorithm for

improved accuracy.  AO runs much faster than MININEC.  The speed

increase is due to code optimization, special analysis modes,

and extensive use of assembly language.

This document covers general aspects of AO.  See OPT.DOC

for information about its optimizing capabilities.

---- RUNNING AO ------------------------------------------------

The quickest way to start AO is to give the name of an

antenna file on the command line, for example, AO DIPOLE.  If

you don't know which file you want, type AO and antenna files in

the current directory will be listed.

Select a file by moving the lightbar with the arrow

keys, PgUp, PgDn, Home, or End.  Press Enter to select the

highlighted file.  Alternatively, you can type a filename.  As

you type, the lightbar moves to the first filename that matches

the characters entered.  Press Enter whenever the desired file

is highlighted.

Select the last item ("Other") to enter a file or

directory name not listed.  If you enter a directory containing

antenna files, AO lists them and you can select one.  See the

SET Commands section for information on organizing antenna files

into subdirectories.

Instead of analyzing an antenna file, you can display a

2-D plot file generated previously.  To do this, start AO by

typing AO PLOT.  Plot files in the current directory will be

listed and you can select one.  Alternatively, you can specify

the name of a plot file on the command line, for example, AO

PLOT DIPOLE, to display the plot immediately.

After loading an antenna file, you control AO by making

menu selections with single-keystroke command keys.  Command

keys are highlighted by color in the main menu.  Use lower-case

characters; upper case has a special effect for P and Z.

To return to the Main Menu from a submenu, press Esc.

You also can return by pressing the command key a second time.

You can terminate data entry and exit a submenu with one

1

keystroke by pressing Esc instead of Enter.  You can bypass the

Main Menu and go directly from one submenu to another by

pressing a command key.  Use Esc to hide the Main Menu to view

the whole screen.

The D and N commands output results to a file named RUN.

RUN is an output file created by AO each time it is executed.

RUN holds lengthy results and contains a complete record of the

analysis session.  RUN also contains an image of the antenna

file.  You can examine RUN from within AO with the R command.

You can also use the file later, after quitting AO.  To save a

particular RUN file, you must rename it.  RUN is always

overwritten with new information whenever you execute AO.

To create a summary printout of analysis results, use

PrtSc to print the main screen or the RUN screen.  You can also

quit AO, use your text editor to edit RUN, and print selected

results.

Error messages are highlighted in bright yellow at the

bottom of the screen.  Not all commands are available if an

error is detected in the antenna file (you must correct the

error before proceeding).

---- ANTENNA FILES ---------------------------------------------

Each antenna system is described in a file with the

extension .ANT.  You can create an antenna file with a text

editor before starting AO or you can use the E command to invoke

your editor from within AO to edit a file already loaded.  Once

AO is running, you can load a new antenna file with the A

command.

Antenna files consist of ordinary text characters.  The

format of an antenna file is illustrated by this sample file for

a 3-element Yagi.  Anything following a semicolon is a comment.

Comments can be added freely at the end of a file without using

semicolons.

3-Element Yagi                                  ; Antenna title

Free Space                                 ; Ground description

14.2 MHz                     ; Frequency.  Use kHz, MHz, or GHz

3 wires, feet                ; Number of wires, dimension units

10   -8 -17   0   -8 17   0   1"   ; For each wire: # segments,

10    0 -16.5 0    0 16.5 0   1"     ; XYZ coordinates of ends,

10    7 -16   0    7 16   0   1"                 ; and diameter

1 source                                 ; Number of feedpoints

Wire 2, center  1  0          ; Source location, voltage, phase

0 loads                                              ; Optional

Details of antenna-file entries are explained in

subsequent sections.

2

Antenna-file lines are free-form.  You can position

items anywhere on a line as long as you maintain item order.

Use spaces, commas, or tabs to separate items.  Upper or lower

case can be used.  Most keywords, like "wires", can be singular

or plural.  Blank lines or comment lines can be inserted

anywhere to improve readability.

Dimensions can be specified in feet, inches, meters,

centimeters, or millimeters.  AO looks for just the first two

characters of these keywords, so both American and British

spellings can be used.  You can specify the length-unit

abbreviations given next as well.

Any dimension in the wire list can use its own unit of

length.  Simply append one of the following dimension overrides

to the number (don't leave a space):

' " ft in m cm mm

You can specify wire diameter by entering a wire gauge

prefixed with #, for example, #12.  AO uses American Wire Gauges

for annealed bare copper wire.

---- COORDINATE SYSTEM -----------------------------------------

AO uses the Cartesian coordinate system to refer to

points in space.  X and Y are in the horizontal plane and Z is

height.  If you think of +X as north, then +Y is west.  AO uses

the same azimuth-angle convention as ordinary compass bearings:

0 degrees is along the +X direction (north), and 90 degrees is

along the -Y direction (east).  +Z is up.  The forward horizon

is 0 degrees elevation angle, overhead is 90 degrees, the rear

horizon is 180 degrees, and directly below is -90 degrees.  If

you aim unidirectional antennas in the +X direction, AO

automatically computes forward gain, F/B, and beamwidth.

To get a feeling for the coordinate-system geometry,

imagine facing a 3-element Yagi 5 feet off the ground aimed at

you.  Assume that the center of the driven element is at X = 0,

Y = 0, Z = 5.  The main lobe of the beam points directly at you,

in the +X direction.  The elements extend in the -Y direction to

your left, and in the +Y direction to your right.  The director

has a +X coordinate, while the reflector is in -X territory.

Positive azimuth angle is to your left (clockwise as viewed from

above), and positive elevation angle is above your head.

---- WIRES, SEGMENTS, AND PULSES -------------------------------

A wire is always straight in AO.  You model a bent wire

by connecting two or more straight wires.  Wires are considered

to be connected when ends have identical coordinates.  For

example, each loop of a cubical quad antenna is described by

four individual wires whose endpoints lie at four points.  Thus

you model a 2-element quad with eight wires, even though a real

3

antenna has just two continuous wires strung on the spreaders.

You can model a Yagi element using tapered, telescoping tubing

with several connected wires of different diameter.

Connections are allowed only at wire ends.  Wires that

cross or terminate at midpoints of other wires are not

considered to be connected.  To model such geometries, subdivide

the wires into shorter wires with coincident endpoints.  When

modeling antennas over ground, wire ends with Z = 0 are always

considered to be grounded.  You can define wires in any order in

the antenna file.

Wires are divided into segments for analysis.  You can

let AO segment wires automatically or you can specify the number

of segments for one or more wires yourself.  Increasing the

number of segments generally improves accuracy, but analysis

then takes longer.  Using between 4 and 40 segments per halfwave

of wire provides useful results.

AO uses the wire segmentation to model conductor current

in sections called pulses.  AO models current as uniform within

each pulse.  Pulses constitute the entire representation of wire

current for the antenna model.  They're distributed as follows:

Pulses are centered at segment junctions and have the same

length as segments.  A pulse is placed at each wire junction and

overlaps onto two wires; a junction of n wires has n-1 pulses.

A half-length pulse is placed at each grounded wire end.  No

pulse overlaps the last half-segment of an unconnected wire end.

The maximum number of pulses is limited only by

available memory.  See the Memory section for details.  The

number of wires is limited only by the number of pulses.

To check that you're using enough pulses, increase the

number of wire segments and reanalyze to see whether results

change significantly.  Until you develop a feeling for the

segmentation necessary under various conditions, this

convergence check is very important.

Whenever you change the analysis frequency with the

Frequency Menu, the number of segments is not altered.  If you

increase frequency by a large amount, the segmentation density

may no longer be adequate.

For valid results, segments should not be too short.

The length of each segment should be greater than .0001

wavelength and greater than the wire diameter.  AO displays a

warning message when either of these limits is exceeded.

As AO builds an internal representation of the antenna,

it may reorder, subdivide, or ignore wires due to special

processing described in subsequent sections.  The Wires and

Pulses section of the RUN file reflects this internal

organization.  There may not be a one-for-one correspondence

between the wires listed and those in the antenna file.

4

---- AUTOMATIC WIRE SEGMENTATION -------------------------------

You can control wire segmentation from the Options Menu

by specifying the number of segments allocated per half-

wavelength of wire.

AO allocates 10 segments per halfwave by default.  This

gives good results in most cases.  For quicker results, use

sparse segmentation, for example, 5 segments per halfwave.  For

increased accuracy, use dense segmentation, like 20 segments per

halfwave.

Until you're familiar with the segmentation necessary

for various types of models, it's important to test convergence

by increasing segmentation density until results no longer

change significantly.  10 segments per halfwave is adequate for

most models, but not for all.

You can disable automatic segmentation by setting the

segmentation density to 0 in the Options Menu.  AO then

allocates the number of segments specified for each wire in the

antenna file.

When automatic segmentation is used, the number of

segments specified on wire lines still affects segmentation in

two ways.  When the number is positive, automatic segmentation

allocates for the wire at least the number of segments

specified.  This feature can be used to ensure adequate

segmentation for special cases like embedded transmission lines.

When the number of segments is preceded by a minus sign, AO

disables automatic segmentation for the wire and allocates the

number of segments specified.  This feature can be used to

allocate fewer segments than would be allocated automatically.

With guy wires, for example, sparse segmentation of the guys and

automatic segmentation of the antenna can be used to reduce

model size without loss of accuracy.

Except for special cases, it's best to leave automatic

segmentation enabled and specify one segment for each wire in

the antenna file.  This provides the widest possible range for

automatic segmentation.  Whenever you encounter a file with a

number other than 1 specified, you'll know that special

precautions are being taken.

---- AUTOMATIC SEGMENT-LENGTH TAPERING -------------------------

Whenever the lengths of two connected segments differ by

more than a factor of about 2, you may get inaccurate results.

All segments on a wire have the same length.  Problem junctions

typically occur when a short wire with one short segment is

connected to a much longer wire with long segments.  Allocating

more segments to the longer wire will fix the problem, but this

can waste a lot of segments.

5

AO can automatically subdivide a long wire into several

single-segment shorter wires with geometrically increasing

(tapered) segment-lengths.  The shortest wire is placed at the

problem junction.  The length of succeeding wires is gradually

increased until it is about the same as the original segment

length.  A single wire with multiple segments is then used for

the remainder of the span.

When automatic segment-length tapering is enabled, AO

analyzes the entire antenna for problem junctions.  All

junctions are corrected.  After the process completes, no

connected segments will differ in length by more than a certain

ratio.  The ratio is 2 by default.  See the SET Commands section

to change this value.

Automatic segment-length tapering normally should be

enabled.  It can be disabled from the Options Menu for testing.

This feature works even when automatic segmentation is disabled.

---- PULSE IDENTIFICATION --------------------------------------

Sources and loads must be located at pulses (segment

junctions).  A particular pulse can be identified by specifying

its pulse number.  Pulse numbers are assigned by AO and are

listed in the Wires and Pulses section of the RUN file.  To

specify pulse number, use the following phrase following a

number-of-sources or number-of-loads line:

Pulse 1

Pulse number 1 is used here for illustration.  The

keyword "pulse" can use upper or lower case (in many contexts,

this keyword can be omitted entirely).

Sources and loads usually are located at the center or

end of a wire.  When this is the case, there is an easier way to

identify a pulse.  In place of the pulse-number phrase, use one

of the following:

Wire 1, Center

Wire 1, End1

Wire 1, End2

Wire number 1 (the first wire specified) is used here

for illustration.  End1 refers to the first end specified on the

wire line, and End2 to the second.  You can use upper or lower

case.  The comma is optional.

When a wire center is specified but no pulse is located

there, AO increments the number of segments for the wire by one

to provide a center pulse.  This trick almost always works, but

it may fail if the wire undergoes automatic segment-length

tapering at just one end.  If AO cannot fabricate a center

pulse, it selects the pulse nearest the center and displays a

warning message.  Usually a change in segmentation density will

6

provide a center pulse.  For a permanent fix, divide the wire

into two equal-length wires and specify a wire end.

If a pulse is specified at an unconnected wire end, AO

displays an error message.  No pulse exists at a free wire end

and there is no current-return path for a source or load.  You

can specify a grounded wire end since ground provides the return

path.

In addition to being highlighted in the geometry

display, source and load pulses are identified in the RUN file

in the Wires and Pulses section and in the Currents section.

---- SOURCES ---------------------------------------------------

You can apply a source of power at any pulse.  Think of

the wire as cut in two at the source location and a feedline or

RF generator inserted in the gap.  At a multiwire junction,

think of the specified wire as fed against the junction.

Antenna excitation in AO uses an infinitely small feedpoint gap.

If you try to imagine feedpoint gaps with finite separation,

you'll become hopelessly confused.

You must take into account the feedpoint location when

manually segmenting driven wires because an antenna can be

excited only where a pulse is located.  In the Yagi example

given earlier, the driven element uses an even number of

segments.  This results in a pulse at the exact wire center at

the junction of the two middle segments.  Specifying an odd

number of segments for the driven element would permit only an

off-center feedpoint.  Automatic segmentation normally takes

care of this problem automatically.

You can specify either voltage or current sources.

Voltage sources have zero source impedance, while that of

current sources is infinite.  To specify current sources, put

"current" anywhere on the number-of-sources line.  Current is

specified in amperes and degrees.  Current feed is very handy

when designing phased arrays.  Many phased-array designs specify

element-current ratios rather than the complex phasing systems

required to achieve those ratios.  Unless you're interested in

absolute near- or far-field values, it makes no difference

whether you use a voltage source or a current source for single-

source models.

When you omit source magnitude and phase, 1 volt (or 1

amp) and 0 degrees are used.  This default is convenient for

single-source models.

You can change source characteristics either by editing

the antenna file or by using the Sources command.  The latter

does not make any changes to the antenna file and does not cause

recalculation of the mutual-impedance matrix.  This command is

handy when experimenting with phased-array systems.  The first

ten sources can be modified from the Sources Menu.

7

When modeling designs with multiple sources, you may get

negative values for input resistance.  This simply means that

power is flowing into a source rather than away from it.  The

power originates at other sources and is coupled to the

negative-resistance source by the near field.  This situation

often occurs for multiple-source models and is normal.  However,

negative input resistance for a single-source model is a sure

sign of improper segmentation.

MININEC normally does not allow a source (or load) to be

defined on the first wire of a junction of three or more wires.

However, AO automatically reorders wires to accomodate this

condition.

The number of sources is limited only by the number of

pulses.

---- LOADS -----------------------------------------------------

AO can model antennas with lumped loads.  Loaded

antennas include multiband Yagis, dipoles, and verticals with LC

traps, shortened dipoles and whips with loading coils, and

traveling-wave antennas with resistors (rhombics, V-beams, and

Beverages).  Loads must be located at pulses.  You can subdivide

a wire into two wires to accomodate a load where no pulse exists

since a pulse is placed at every wire junction.  You can

identify loads by pulse number or by specifying a wire center or

end.

AO provides several ways to specify loads.  It's easiest

to define simple loads as RLC loads.  Here's an example of three

RLC loads:

3 loads

Wire 1, Center   27 pF  4.7 uH  3 ohms

Wire 2, End2     4.7E-6 H  Q=300

Wire 7, Center   600 ohms

This load syntax specifies RLC component values for

parallel traps.  R is in series with L, and C is in parallel

with RL.  For simpler loads, use fewer components.  For example,

the second load defines a lossy inductor, while the third

defines a resistor.  Component values not specified are set to

0.  (When just C is specified, R and L are removed from the

circuit to avoid shorting C.)  Use Q to automatically determine

the series R for a lossy inductor.  When L is not specified, Q

determines the parallel R for a lossy capacitor.  (Once R is

determined from Q, it does not vary with frequency.  Q is

defined at the file frequency.)

The RLC syntax can model a lossy trap, a lossless trap,

an inductor, a lossy inductor, a capacitor, a lossy capacitor,

or a resistor.  AO recognizes the following units:  pF, F, nH,

uH, H, and ohms.  You can use upper or lower case.  Units must

be separated from values by at least one space.  You can specify

8

RLC components in any order on a line.  You can separate entries

by spaces, commas, or tabs.

Internally, AO contains two types of load models,

impedance loads and Laplace Transform loads.  AO translates RLC

loads into Laplace Transform loads.  The RLC syntax is adequate

for all common loads.  For special cases, you can specify

impedance loads or Laplace Transform loads directly.

An impedance load is a resistance in series with a

reactance.  Unlike RLC loads, the reactance does not vary with

frequency.  Here is a syntax example for impedance loads:

2 loads

Impedance Loads

Wire 2, End2   740  122

Wire 4, Center  44

"Impedance Loads" is not a keyword.  AO recognizes an

impedance load whenever this line begins with anything other

than "Laplace", "Wire", or "Pulse".  This lets you use a

descriptive phrase (for example, "Capacitive Reactance") to

invoke an impedance load and simultaneously annotate the file.

The two values on the wire line are resistance and reactance

(resistance first), both in ohms, without units.  When the

reactance value is omitted, it is assumed to be zero.

Laplace Transform loads permit complex networks to be

modeled.  A Laplace Transform is a special polynomial

representation of a lumped circuit.  Laplace Transforms are too

complicated to explain here, but the following example

illustrates the syntax:

2 loads

Laplace Transform

Wire 1, End2                             ; First load location

0   8.2E-6                     ; Numerator coefficents s^0 s^1

1   0   4.92E-16        ; Denominator coefficients s^0 s^1 s^2

Wire 4, End1                            ; Second load location

0   8.2E-6         ; The second load is identical to the first

1   0   4.92E-16

The maximum transform order is 6.  Enter coefficients in

order, starting with s^0.  Magnitudes smaller than 5.8775E-39

cannot be used.  Higher-order zero coefficients need not be

entered, as illustrated.  When calculating coefficients, use

ohms, Henries, and Farads.  Consult a textbook on circuit

analysis for information on Laplace Transforms.

A load coincident with a source appears in series with

the source.  The number of loads is limited only by the number

of pulses.

9

---- FREE-SPACE SYMMETRIC ANTENNAS -----------------------------

AO provides a special technique for modeling symmetrical

antennas in free space.  Twice the usual number of pulses are

available with this technique.  In addition, when the same

number of pulses is used, the analysis runs up to three times

faster.  Free-space symmetric mode should be used whenever

possible to speed computation.  (Simple, single-wire models with

less than 90 pulses run faster using regular analysis mode.)

In free-space symmetric mode, AO mirrors the antenna

horizontally about the X-Z plane.  Wires or portions of wires in

the -Y half-space are ignored.  A mirror-image of each wire and

wire section in the +Y half-space is substituted.  The direction

of instantaneous current flow in image wires is set by mirror-

imaging the flow, then reversing it.  For example, the image of

a horizontal wire parallel to the Y axis will have in-phase

current, while the image of a vertical wire parallel to the Z

axis will have out-of-phase current.

The complete antenna often is defined in free-space

symmetric files even though everything in the -Y half-space will

be ignored.  This makes it simple to modify the file later for

analysis over ground.  However, you can save some typing by

entering just the +Y half for large, symmetric models.

When n pulses have been allocated for half of a free-

space symmetric antenna, n more effectively are being utilized

by the antenna image.  This yields a total of 2n pulses for the

complete model (except for pulses located in the image plane;

these pulses are never imaged).

To invoke free-space symmetric mode, just add the

keyword "symmetric" after "free space" on the second line of the

antenna file.  (You don't have to remember how many ms are in

symmetric; AO looks only for "sym".)  If the file identifies

sources and loads by specifying wire centers or ends, nothing

else needs to be changed.

However, if sources or loads are identified by pulse

number, the numbers must be changed.  Pulse allocation is

different for free-space symmetric models since only half of the

antenna is actually modeled.  Examine the RUN file to determine

new pulse numbers.  The notation "Image" in the Wires and Pulses

list indicates the connection of a wire to its symmetric mirror-

image at the Y = 0 image plane, where a pulse is always placed.

If necessary, AO increments the number of segments for a wire by

1 to fabricate a pulse at Y = 0.

Sources and loads specified at -Y coordinates are

ignored, while those with +Y coordinates are mirrored into the

-Y half-plane.  Sources and loads located at the Y = 0 image

plane are modeled normally.  If a wire lies in the image plane

of a free-space symmetric model, an error results.

10

---- GROUND ----------------------------------------------------

AO models an antenna over ground whenever the second

line of the antenna file begins with anything other than "free

space".  You can put a short description of ground

characteristics on this line to cause modeling over ground and

to annotate the file at the same time.

You can use simple default ground characteristics or you

can define an elaborate ground model in the antenna file.  You

can define up to ten concentric ground zones, each with its own

dielectric constant, conductivity, radius, and height.  Ground

zones can be used to model antennas on hilltops (steep drop-offs

only), in swamps or marshes, or mounted on vessels above the

water line.  Ground zones are centered at the origin.  The outer

zone extends to infinity.  A perfect-conductivity groundplane

extending to infinity can be defined as well.

You can model a ground screen composed of radial wires.

When you specify a ground screen and one ground zone, soil

characteristics are assumed to be uniform.  For two or more

zones, zone 1 defines the soil under the screen and zone 2

begins where the radials end.  This model is useful when the

soil under the ground screen has been chemically treated to

increase conductivity and exhibits different properties from the

earth beyond.  The effective RF impedance of a ground screen is

combined with that of the earth beneath.  This impedance is used

to calculate the ground-reflection factor for the far-field

tabulations and patterns.  The ground-screen model has high

accuracy only for dense screens employing more than 100 radial

wires.

The height of the first ground zone is always zero.  You

can use the height of additional ground zones to roughly

approximate elevation variation in local terrain.  However, this

technique will not model sloping ground accurately, even if many

stairstep ground zones are used.  Moreover, when the height

drops from one zone to the next, diffraction from the cliff-edge

formed is not modeled.  When the height increases, blockage of

radiation from the wall formed is not modeled.

The antenna need not be located at the center of the

ground zones.  The ground zones are concentric with the origin

but you can place the antenna anywhere.  You might do this to

model reflection from a small lake near the antenna, for

example.

You should exercise caution when modeling antennas over

ground.  AO uses ground characteristics only to determine the

ground-reflection factor for the far-field tabulations and

patterns.  It uses a perfect-conductivity groundplane when it

calculates wire currents.  This implies that ground-current

losses are not accounted for.  While these losses normally are

negligible for horizontal antennas higher than about 0.2

wavelength, they can be significant for low horizontals and for

verticals fed against poor ground systems.  In these cases, the

11

calculated gain will be too high and the calculated impedance

too low.  You can add a resistive load at the feedpoint to

simulate ground-current losses.

Often the most suitable reference for an antenna over

ground is not a dipole or isotropic radiator in free space, but

a dipole, monopole, or single array element substituted for the

antenna under analysis.  The reference antenna should be modeled

with the same frequency, polarization, and ground conditions as

the antenna under study.

Keep in mind that dBd and dBi gain figures refer to

free-space reference antennas, not to reference antennas in the

same environment as the antenna under analysis.  At small

elevation angles, expect to see negative gain values over ground

for antennas that exhibit gain in free space.  In particular,

vertical antennas have very little response at elevation angles

near the horizon unless they radiate over salt water.  The same

is true for horizontal antennas over all types of ground unless

they are very high.  Conversely, expect up to 6 dB gain over

free-space values at lobe maxima in the elevation plane.  Gain

occurs where the ground-reflection wave adds constructively with

the direct wave.

It takes up to three times longer to analyze an antenna

over ground as in free space.  When optimizing a design, it's

quicker to do preliminary runs in free space and leave final

optimization over ground for last.

Here's an example of an antenna file using a simple

ground model:

Quarter-Wave Vertical

Ground Mounted

7.15 MHz

1 wire, feet

1   0 0 0   0 0 33   1"

1 source

Wire 1, End1

When the second line of an antenna file does not begin

with "free space", AO models the antenna over ground.  In this

example, no additional ground information follows the second

line, so a single ground zone with default ground

characteristics is used.  Default characteristics can be

predefined using a DOS SET command as follows:

SET GND=D C

where D = dielectric constant and C = conductivity in mS/m.  AO

uses average ground constants (D = 13 and C = 5) if GND has not

been defined.  To define a perfect-conductivity groundplane as

the default, use SET GND=0.

Here's an example of an elaborate ground model involving

several ground zones:

12

Quarter-Wave Vertical

Ground Mounted

3 zones, 120 radials #18  ; # of zones, # of radials, wire gauge

13 5    0 100   ; Zone #1 diel. const., conduct., height, radius

17 7  -50 500   ; Zone #2

19 2 -200       ; Last zone goes to infinity so no radius needed

7.15 MHz

1 wire, feet  ; These units apply to the ground dimensions above

1   0 0 0   0 0 33   1"

1 source

Wire 1, End1

You can specify up to 10 zones.  The height of the first

zone is always 0.  In this example, 100 feet from the origin the

ground drops 50 feet.  It then drops an additional 150 feet at a

500-foot radius.  When you specify a single ground zone, you can

omit height and radius.

The radial specification is optional.  You can omit

everything after the word "zones" (which can be singular or

plural) when radials are not modeled.  Radial length is equal to

the radius of the first ground zone.  When you specify radials

and a single ground zone, the zone radius value determines

radial length and the zone itself extends to infinity.

To specify a perfect-conductivity groundplane, the zone

line should read "0 zones" and there should be no additional

ground lines.

Here's a table of U.S. ground constants:

ÿ                 Earth                   Diel  Cond    Ground

ÿ                Surface                 Const  mS/m   Quality

-------------------------------------- -----  ----  ---------

Salt water                               81   5000  Excellent

Fresh water                              80    1

Pastoral, low hills, rich soil           20   30    Very Good

(Dallas TX and Lincoln NE)

Pastoral, low hills, rich soil           14   10

(OH and IL)

Flat country, marshy, densely wooded     12    7.5

(LA near Mississippi River)

Pastoral, medium hills and forestation   13    6

(MD, PA, NY, except mtns & seacoast)

Pastoral, medium hills and forestation,  13    5    Average

heavy clay soil (central VA)

Rocky soil, steep hills (New England)    14    2    Poor

Sandy, dry, flat (coastal)               10    2

Urban and industrial areas (average)      5    1    Very Poor

Urban and industrial areas (worst)        3    0.1  Extr Poor

A detailed map of U.S. ground conductivities appears in

recent editions of the ARRL Antenna Book on page 3-3.  As the

map indicates, ground conductivity has much greater local

13

variation than the simplified table above suggests.  Ground

constants don't affect horizontally polarized antennas much

(except for high-angle radiation and the impedance/losses of

very low antennas), but they critically affect the performance

of vertical arrays.

---- WIRE-CONDUCTIVITY LOSSES ----------------------------------

AO models losses due to wire conductivity and skin-

effect.  You can specify conductor material on the number-of-

wires line like this:

1 copper wire, inches

AO recognizes the following materials:

Silver          1.59E-08      Pure silver

Copper          1.7241E-08    100% IACS

Aluminum        2.655E-08     Pure aluminum

6063-T832       3.25E-08      Aluminum alloy

6061-T6         4.01E-08      Aluminum alloy

Brass           6.4E-08       Yellow brass (35% zinc)

Phosphor Bronze 1.1E-07       Use two words (5% tin)

Steel           7.2E-07       Stainless steel type 302

Volume resistivity values in ohm-m are given here for

reference.  You can specify resistivity or conductivity directly

as follows:

1 resistivity 5.7E-08 wire, inches

1 conductivity 53 wire, inches

Conductivity is specified in IACS units.  IACS stands

for International Annealed Copper Standard.  A material's IACS

value is its conductivity expressed as a percentage of that of

annealed copper.  Pure copper is 103.06 IACS.

For ferromagnetic material like iron or steel, you can

specify permeability as follows:

1 resistivity 5.7E-08 permeability 1.5 wire, inches

1 conductivity 53 permeability 2.3 wire, inches

You can abbreviate resistivity and permeability to res

and per.

AO models all antenna wires using the wire material

specified on the number-of-wires line.  To specify a different

material for a particular wire, append a wire-material phrase to

the wire line after wire diameter.

When present, AO displays wire losses in dB.  Load

losses also are displayed in dB.  When both kinds of losses are

present, AO displays the total loss in dB and the individual

losses in percent of input power.  AO also displays antenna

14

efficiency in percent.  This figure includes the effects of both

wire and load losses.

---- STANDING-WAVE RATIO ---------------------------------------

AO calculates SWR in two different ways.  The first way

gives SWR in a 50-ohm feedline attached to the antenna

feedpoint.  This SWR value is useful for single-frequency

analysis (use the Options Menu to change the 50-ohm default

value).  The alternate SWR calculation uses the complex

conjugate of input impedance at one frequency as the SWR

reference impedance.  This calculation reveals SWR variation

with frequency once the antenna has been perfectly matched at a

particular frequency.

The alternate SWR value is displayed during automatic

frequency sweep (it's also displayed during regular analysis

after the frequency has been changed).  This SWR value results

from a perfect match to the antenna input impedance at the

middle of the sweep range using an idealized matching network

(for unswept analysis, the match is done at the first frequency

analyzed after loading the file or after frequency sweep).

The matching network is modeled as a fixed series

reactance followed by a broadband transformer.  The series

reactance cancels input reactance, while the transformer matches

input resistance to the feedline impedance.  The matching

network is idealized:  The series reactance and transformer

impedance ratio do not vary with frequency.

The alternate SWR value gives the inherent SWR-bandwidth

properties of the antenna independent of any particular

narrowband matching network.

To keep things simple, SWR is not computed for antennas

with multiple sources.

---- SYMBOLIC DIMENSIONS ---------------------------------------

Regular antenna structures often have identical

coordinate values for several wires.  Here's an example:

2-Element Cubical Quad

Free Space Symmetric

24.94 MHz

8 wires, inches

1   -35.5 -64.75 -64.75   -35.5  64.75 -64.75   #12

1   -35.5 -64.75 -64.75   -35.5 -64.75  64.75   #12

1   -35.5 -64.75  64.75   -35.5  64.75  64.75   #12

1   -35.5  64.75  64.75   -35.5  64.75 -64.75   #12

1    35.5 -61.25 -61.25    35.5  61.25 -61.25   #12

1    35.5 -61.25 -61.25    35.5 -61.25  61.25   #12

1    35.5 -61.25  61.25    35.5  61.25  61.25   #12

1    35.5  61.25  61.25    35.5  61.25 -61.25   #12

15

1 source

Wire 5, center

Unless your text editor provides a search-and-replace

function, it can be tedious to make changes to such a file.  AO

provides a simple solution to this problem with symbolic

dimensions:

2-Element Cubical Quad

Free Space Symmetric

24.94 MHz

8 wires, inches

seg = 1

sp = 35.5

ref = 64.75

de = 61.25

diam = #12

seg   -sp -ref -ref   -sp  ref -ref    diam

seg   -sp -ref -ref   -sp -ref  ref    diam

seg   -sp -ref  ref   -sp  ref  ref    diam

seg   -sp  ref  ref   -sp  ref -ref    diam

seg    sp  -de  -de    sp   de  -de    diam

seg    sp  -de  -de    sp  -de   de    diam

seg    sp  -de   de    sp   de   de    diam

seg    sp   de   de    sp   de  -de    diam

1 source

Wire 5, center

Here, the symbols seg, sp, ref, de, and diam are defined

and then used in place of actual dimensions.  Geometry changes

become easy.  For example, to change the reflector size, simply

edit the single line that defines ref.

Symbol names cannot begin with a digit but they can be

any length.  You can define up to 50 symbols.  Wire symbols are

defined after the number-of-wires line and before the first

wire.  You can use symbols for sources and loads, too.  Define

these symbols just after the number-of-sources and number-of-

loads lines (don't include load units when you define load

symbols; put the units on the load line).  It's not necessary to

use a symbol for every dimension as shown above, nor must you

leave spaces around the =.  Any symbol can be preceded by a

minus sign.  If you attempt to use an undefined symbol, AO

displays an error message.

The optimizer requires that symbols be used.

NOTE:  Unlike everything else in AO, symbol names are

case-dependent.  The symbol LENGTH is not the same as the

symbols Length or length.

16

---- SYMBOLIC EXPRESSIONS --------------------------------------

For certain antenna geometries, it's very handy to use

expressions in symbol definitions.  For example, you might do

something simple like this:

w = x + 5

You might also do something complicated like this:

w = (x + y / (z - 17')) * 23

Symbols like x, y, and z must have been previously

defined.  You can use dimension overrides (like 17') with

numbers, but not with symbols (z' is not allowed).  Unless you

use parentheses, * and / are done before + and -.  You can

separate expression items with zero or more spaces or tabs, but

don't use commas within expressions.  Numbers can use scientific

notation (12300 can be represented as 1.23E+04, 1.23e4, etc.,

while .000123 is 1.23E-04, 1.23e-4, etc.).  Unary plus and minus

are supported.  Items like #12 indicate wire gauge.

In addition to the + - * / operators, AO recognizes

sine, cosine, and square root functions.  Sines and cosines are

very handy for defining certain antenna geometries.  For

example, here's the geometry definition for an 80-meter, three-

square array:

x = 40'

a = SIN(30) * x

b = COS(30) * x

1    x  0 0    x  0 61'   1"

1   -a -b 0   -a -b 61'   1"

1   -a  b 0   -a  b 61'   1"

Here, AO computes the sine and cosine of 30 degrees, a

constant.  You can also use an expression as a function

argument, for example, SIN(x + y).  You can use upper or lower

case for function names.  Use the SQR operator for square roots.

Expressions are very useful when optimizing a design.

They can be used to maintain antenna geometry while allowing a

single independent variable to determine structure size.

Expressions can't be used in place of symbols on wire

lines; they're allowed only in symbol definitions.  However, you

can precede any symbol with a minus sign on wire lines, as

illustrated above.

---- SHIFT AND ROTATE ------------------------------------------

You can shift and rotate an object comprised of one or

more wires.  You can use symbols for the shift and rotate

amounts.

17

To shift an object, put a shift command immediately

before the wires that define the object.  A shift command looks

like this:

Shift X 30

X can be the letter X, Y, or Z and indicates the axis

along which the shift occurs.  Here, the shift amount of 30 is

in the units defined on the number-of-wires line.  You can

append dimension overrides (' " ft in mm cm m) to use different

units.  You cannot use an expression for the shift amount; it

must be a constant or a symbol (however, a symbol can be

preceded by a minus sign).  You can use upper or lower case, and

you can use spaces, tabs, or commas to separate line items.

To designate the end of a shifted object, put this

command immediately after the last wire in the object:

Shift End

All wires between the shift and shift-end lines will be

shifted.  Often you won't need a shift-end command.  If no wires

follow the object to be shifted, you can omit the shift-end

command.

To shift an object along more than one axis, use

multiple shift lines or simply add more items to a single shift

line like this:

Shift X 30, Y -18, Z 55

X, Y, and Z can appear in any order on a shift line.

The commas are optional; they're used here just to improve

readability.  Shift End terminates all shifts (this means that

shifts cannot be nested).

To rotate an object, do this:

Rotate X 30

This rotates the object 30 degrees about the X axis.

Positive rotation about the X axis is defined as +Z rotated

toward +Y.  For Y, it's +X toward +Z, and for Z it's +Y toward

+X.  Positive rotation is the prior axis rotated toward the next

axis in the circular sequence XYZXYZ....  If you can't remember

this convention, just try a positive number.  If the object

rotates the wrong way, add a minus sign.

To rotate an object about more than one axis, use

multiple rotate lines or simply add more items to a single

rotate line.  Unlike shifts, the order of rotates affects final

geometry.  Rotates are executed in reverse file order; the

18

rotate command nearest the object wires is done first.  If you

use multiple rotates on one line, the last one is done first.

To define the end of a rotated object, use this command:

Rotate End

Rotate end terminates all rotates (this means that

rotates cannot be nested).

You can mix shifts and rotates.  The commands are

executed in reverse order of file appearance.  Order matters.

Usually it's most convenient to define the antenna at the

origin, rotate it there, and then shift it.

To rotate an object about an axis and then rotate

another object a different amount about the same axis, you don't

need an intermediate rotate-end command.  For example, here's

how to define an inverted-V dipole with a 120-degree apex angle

and an apex height of 60 feet:

Inverted V

Over Ground

7.15 MHz

2 wires, feet

Shift Z 60

Rotate X 30

1   0 0 0   0  33 0   #12

Rotate X -30

1   0 0 0   0 -33 0   #12

1 source

Wire 1, end1

The second rotate just modifies the rotation value

already in effect; no rotate end is needed.

Shifts and rotates let you define antennas with oblique

wires using actual wire lengths.  You don't need to use

trigonometry to calculate the coordinates of wire ends.

---- POLARIZATION COMPONENTS -----------------------------------

AO normally obtains results using the total

electromagnetic field.  This is equivalent to using the

polarization component (possibly elliptical) that maximizes

electromagnetic-field magnitude at each azimuth and elevation

angle.  The total field provides results that are independent of

wave polarization.  Since polarization is variable for

ionospherically propagated signals, the total field provides a

realistic way to characterize HF-antenna performance.

However, sometimes it's important to isolate a

particular polarization component.  You can do this with the

Options Menu by specifying one of the following:

19

Total         Total Field

H             Horizontal Component

V             Vertical Component

RC            Right-Circular Component

LC            Left-Circular Component

Major         Major-Axis Component

Minor         Minor-Axis Component

Ellipticity   Polarization-Ellipse Circularity

At each azimuth and elevation angle, the major-axis

component is the linear-field magnitude at the polarization tilt

that maximizes the field.  The minor-axis component is the

linear-field magnitude at the polarization tilt that minimizes

the field.  The minor-axis component is always zero for linearly

polarized antennas.

AO treats ellipticity as if it were a field component,

but it's really the ratio of minor- to major-axis components

expressed in dB.  Ellipticity is -infinity dB at all angles for

linearly polarized antennas and 0 dB at angles where

elliptically polarized designs have perfect circularity.

When you select a field component, all analysis results,

including beamwidths and sidelobe levels, refer to that

component.

AO saves all field components internally during

analysis.  You can plot any component without recalculation of

the mutual-impedance matrix.  To compare 2-D polarization

patterns, use the upper-case P plot command to generate a new

plot file after selecting a different polarization component.

Then use the overlay or compare commands while viewing the plot.

---- NEAR FIELD ------------------------------------------------

AO can calculate the electric and magnetic fields

produced by an antenna at any point in space.  The field close

to the antenna is called the near field.  This field is quite

different from the far field that characterizes propagated

electromagnetic waves.  Near-field intensities are useful for

EMI studies, TVI minimization, and RF-hazard analysis.

AO computes the near field over a 3-D grid of points.

Enter starting, ending, and step (increment) values for the

coordinates to be varied.  Set step to 0 for coordinates that do

not vary.  (Although the peak-field value appears on the

Z-component line, it applies to the entire field vector.)

When AO calculates the near field over ground, it uses

perfect-conductivity ground regardless of the ground

characteristics defined in the antenna file.

20

---- FAR FIELD -------------------------------------------------

The Far Field command tabulates absolute or relative

far-field response in the RUN file.  For the absolute far field,

AO tabulates the real and imaginary parts of the horizontal and

vertical field components in volts/meter.  (At nonzero elevation

angles, the "vertical" component is the component upward-

perpendicular to a line to the antenna.)  You specify distance

from the antenna and input power for the absolute far-field.

For the relative far field, AO tabulates the magnitudes of all

polarization components in dBi or dBd; distance and power are

not relevant.

The absolute far field is useful for signal-coverage

studies and interference-level analysis.  (NOTE:  AO calculates

only the space wave and the ground-reflection wave; it does not

calculate the surface wave.)  The relative far field can be used

to obtain response at angular resolutions not available in the

far-field plots.  It also provides a convenient way to output

pattern data to other programs.

Relative far-field values less than -120 dB appear in

the tabulation as -infinity.  This notation suppresses low-level

computational noise and makes the listing easier to read.  See

the SET Commands section to change the threshold value.

---- 3-D GEOMETRY/CURRENT DISPLAY ------------------------------

After loading or editing an antenna file, press V to

view the antenna geometry in 3-D to check for model errors.

Press F1 or any nonfunctional key for a list of viewing

commands.  You can enable or disable display of segments,

sources, loads, wire currents, axes, and annotation.  The

spacebar switches between two viewpoints that can have different

scale and viewing perspective.  You can use one viewpoint for an

overall view of the antenna and the other for close inspection.

Wire currents are displayed after impedance is

calculated.  You can interrupt pattern generation to view the

currents.  Pattern generation resumes without recalculating

prior points.

AO graphs wire currents directly on the wires in the 3-D

geometry display.  You can display currents either as phasors or

using magnitude only.  For phasors, the distance from a wire to

the current trace represents magnitude, while the rotation angle

of the trace around the wire represents phase.  Wires with

traveling-wave currents exhibit non-planar current traces that

become spirals for pure traveling waves.  Use the W key to sight

down a wire axis to examine the circularity of a spiral.  You

can toggle between phasor currents and magnitude-only currents

with the M key.  Use magnitude-only to visualize current

distribution in complex antennas.

21

Note that current phase changes 180 degrees when you

interchange wire ends in the antenna file.  This ambiguity

affects both tabulated currents and current traces.  If you

define parallel wires consistently, always heading in the same

direction from End1 to End2, you can avoid this ambiguity.

---- 3-D PATTERN DISPLAY ---------------------------------------

Press T from the Main Menu or geometry display to

display 3-D polar radiation patterns.  Press F1 or any

nonfunctional key for a menu.  The viewing commands are like

those in the geometry display.

Press R to change angular resolution from the default of

2 degrees.  You can select 2 to 6 degrees for any model and 1

degree for models with X-axis pattern symmetry.  Be sure to take

advantage of X-axis pattern symmetry whenever possible; it can

greatly reduce pattern-calculation time.  Resolution refers to

the elevation-angle increment.  Azimuth-angle resolution varies

automatically with elevation angle (it's the same as elevation-

angle resolution at 0 degrees elevation).  AO saves the current

3-D resolution in AO.INI whenever you save colors or select Save

Options & Settings from the Options Menu.

Press S to slice the pattern at a selected elevation

angle.  This is useful for examining the inside wall of complex

patterns.

Press T to thin a pattern.  This opens up the mass of

white at the center of dense plots.  It also speeds pattern

rotation.  This function works just like a barber's thinning

shears.  As the plot surface contracts, lines are selectively

deleted to reduce pixel density.  Experiment with the thinning

threshold.  Often there's one value that's just right for best

plot appearance.  Thinning a dense plot is particularly useful

before printing it.

Press V to bring up the geometry display.  You can press

V and T to alternate between the two displays.

As in the geometry display, you can use the spacebar to

switch between two viewpoints.  The viewpoints can have

independent perspective and scale.  Two viewpoints are very

useful with 3-D patterns.  For example, you can zoom in with one

viewpoint to examine close pattern detail and switch back and

forth with a second viewpoint showing the overall pattern.

Use the Home and End keys to change the center of

rotation for patterns with high F/B.  This lets you rotate large

patterns without clipping.

22

---- 2-D PATTERN GENERATION ------------------------------------

AO can generate 2-D azimuth and elevation radiation

patterns.  It saves the pattern data in files with the extension

".PLT".  AO searches the data to find beamwidth and maximum

sidelobe level.

By default, AO assumes that antenna patterns are

symmetrical about the X-axis.  This allows it to generate

patterns from 0 to 180 degrees and then mirror-image this data

into the remaining half-plane.  This cuts pattern-calculation

time in half.  For antenna patterns that aren't symmetrical

about the X-axis, use the Options Menu to disable this option to

obtain correct patterns.  Always align antennas with symmetrical

patterns with the X-axis to take advantage of this time-saving

feature.

With the Options Menu you can specify the elevation

angle to use during azimuth-pattern generation and the azimuth

angle for elevation patterns.  For free-space models, the

elevation angle for azimuth patterns defaults to 0 degrees to

obtain on-axis response.

You can generate 2-D patterns with 1-, 2-, or 4-degree

resolution.  4-degree resolution can speed pattern-generation

for large models.  1-degree resolution is useful for complex

patterns.  Use the Options Menu to change 2-D pattern

resolution.

When an azimuth or elevation search finds maximum

response at 0 degrees, AO computes the 3-dB pattern beamwidth.

AO uses interpolation to obtain accurate beamwidth values even

for 4-degree patterns.

When AO finds a sidelobe with amplitude larger than that

of the rear lobe, it displays its level and angle.  This is

useful when manually optimizing an antenna pattern.  It's easy

to get carried away and lose sight of the overall pattern when

optimizing F/B, a measurement that involves just a single point

to the rear of the antenna.  You might achieve 45 dB F/B but

have a sidelobe just 15 dB down at 135 degrees.  The sidelobe

display will alert you to this problem.  (The automatic

optimizer uses a more generalized definition of F/B to avoid

this difficulty.)

When the main lobe occurs at an angle other than 0

degrees, AO does not display beamwidth or sidelobe information.

Instead, it displays the angle of the lobe and its gain.

---- 2-D PATTERN PLOTS -----------------------------------------

You can view 2-D radiation patterns of the current

antenna or you can review plots made previously.  To review a

plot, start AO by typing AO PLOT.  AO will list plot files in

the current directory and you can select one.  Alternatively,

23

you can specify a plot file on the command line, for example, AO

PLOT DIPOLE.  This displays the plot immediately.

AO normally begins by drawing a polar plot of the

azimuth pattern using the log-dB scale (the linear-dB scale is

used when set with the POLAR environment variable).  You can

start differently by entering up to three option letters after

the filename as follows:

E       Elevation pattern

R       Rectangular plot

L       Linear-dB scale

For example, AO PLOT DIPOLE ERL will cause AO to draw

the elevation pattern in rectangular coordinates.  If you later

select a polar plot, it will use the linear-dB scale.  You can

use upper or lower case for option letters.

While viewing a plot, press F1 or any nonfunctional key

for help.  The help panel lists only currently active keys.  For

example, the X and Y keys are listed only in rectangular mode.

The A key turns off overlay and compare modes.  This

lets you examine the patterns of a single antenna more rapidly.

The O and C keys can be used repeatedly to compare multiple

antennas, two at a time.

Polar Plots

AO displays azimuth polar plots with the +X direction to

the top of the screen and +Y to the left.  Elevation plots have

+Z to the top and +X to the right.

AO provides two radial scales for polar plots.  The ARRL

log-dB scale causes lower-level sidelobes to be compressed

toward the center of the pattern.  This emphasizes major-lobe

shape.  The ARRL log-dB scale is widely used in amateur

publications.  It provides a convenient scale to compare the

patterns of antennas you develop with those of existing designs.

It also yields patterns with familiar shapes.  The center of a

log-db plot is -infinity dB, but there isn't much area below -40

dB.

The other polar scale uses linear dB.  This scale cuts

off at -50 dB at the center of the plot.  It provides much more

area between -20 and -50 dB than the log-dB scale.  It's useful

for examining low-level sidelobes that may be hard to see in a

log-dB plot.

The dots forming the radial lines (the ones that are not

multiples of 30 degrees) are spaced every 2 dB.  The dots

forming the circles are spaced at multiples of 1 degree.  These

calibrations allow directivity values to be read with good

accuracy.

24

AO draws a 360-degree elevation polar plot for free-

space patterns.  For patterns over ground, just the upper 180-

degree hemisphere is shown.

The polar plots are perfectly circular on monitors with

standard 4:3 aspect ratio.  If the plots appear elliptical,

adjust your monitor's vertical height control.

Rectangular Plots

Use the R key to select a rectangular plot.  This

coordinate system can resolve small sidelobe detail even better

than a linear-dB polar plot, but the overall pattern shape is

not quite as apparent.  The X axis is azimuth or elevation

angle, and the Y axis is antenna response in dB on a linear

scale.  Use the Y key to change the Y-axis cutoff (lower limit

in dB).  This parameter is always a negative number, but you can

enter it without a minus sign for convenience.  The Y-axis

cutoff is constrained between -1 and -100 dB.  The Y axis is

easiest to interpret when the cutoff value is a multiple of 10

degrees, but AO draws the scale and grid correctly for any

value.  Use the X key to change the X-axis cutoff.  AO rounds

the value entered to the nearest multiple of 10 degrees.

Overlaying Plots

You can overlay a second pattern with the O key.  You

can distinguish the traces and annotations of the two patterns

by their individual colors.  AO substitutes filenames for file

titles when overlaying plots.

Comparing Plots

The C key selects a second plot file for pattern

comparison.  The plot is drawn into a separate screen buffer

(unless only one VGA display page is used).  The screens can be

switched instantly with a single keystroke.  This permits a

precise instantaneous pattern comparison, and can reveal

differences that may be lost in screen clutter in overlay mode.

Generating Plot Data

The p or P commands cause AO to generate and display a

plot file. AO saves the patterns in a file so that you can

review and compare plots later.  A plot file uses the antenna

filename with the extension .PLT.  If you make changes to a

design and press lower-case p, the plot file is overwritten with

new patterns and displayed.  If you press upper-case P, AO

generates a new plot file.  Each new plot file has an

incrementing digit appended to its name.  Use of new filenames

prevents previous plot files from being overwritten.  This

preserves plot files for later comparison.

25

Use the Options Menu to select 1-, 2-, or 4-degree

pattern resolution.

If you just want to view the azimuth response, you need

not wait for elevation data to be generated.  Press Esc while

generating elevation data to draw the azimuth plot.  The plot

file will contain only azimuth data.  Later, if you press p and

wait for data generation to complete, AO adds the elevation data

to the plot file and displays the plot.

AO annotates the electromagnetic-field component on

plots when it is different from the usual total field.  AO

generates plot data with a dynamic range of 100 dB and with a

resolution of .01 dB.  AO annotates the elevation angle on

azimuth plots only when nonzero, and the same holds for the

azimuth angle on elevation plots.

---- SCREEN COLORS AND DEFAULT MENU SETTINGS -------------------

You can customize the screen colors used by AO.  Use the

C key to change colors in the main screen and the optimizing

screen.  Press F5 to change colors in other screens.  It's easy

to experiment with various color schemes.  To make changes

permanent, use the S command from one of the color menus or

select Save Options & Settings in the Options Menu.  These

functions write codes for the current color sets into the

initialization file AO.INI.  Whenever AO finds AO.INI in the

current directory on start-up, it sets all colors accordingly.

AO.INI also contains codes for most menu settings.  If

AO.INI is present when you start AO, all menus are initialized

to the saved AO.INI settings.  To customize AO to your way of

working, change the menus to your preferred settings and then

save them with Save Options & Settings.

To reset colors and menu settings to the defaults,

simply delete AO.INI.

---- PRINTING THE SCREEN ---------------------------------------

AO prints text and graphics screens to HP LaserJet/

DeskJet printers and to Epson-compatible, dot-matrix printers.

Press PrtSc to print any screen.  See the SET Commands section

for details on configuring AO for your particular printer.

Unless you're in LaserJet landscape mode, AO does not

eject the page after printing a screen.  This lets you print two

screens on one page (24-pin prints are too big to allow this).

You might print two plots for comparison or print a graphics

screen accompanied by a listing of the antenna file.  To list

the antenna file, press PrtSc while viewing its image in the RUN

file.  Alternatively, you can save the current design to PRN or

LPT with the Save Menu in the optimizer.

26

To correctly print extended-ASCII characters like the

degree symbol, set your printer for the PC-8 symbol set.

---- GRAPHICS IMAGE FILES --------------------------------------

AO can save the image of any graphics screen in the .PCX

file format.  This feature lets you add AO graphics to desktop-

publishing and word-processing documents.  If your computer has

a fax/modem, you can fax AO images.

Press F9 to generate a .PCX file.  The output filename

is the antenna filename with the extension .PCX.  Whenever you

press F9 again, an incrementing digit is appended to the

filename and another file is created.  You can control image

size and centering with a DOS SET command.  See the SET Commands

section for details.

---- ABORTING CALCULATIONS -------------------------------------

You can abort calculation by pressing the Esc key.

Since the most fundamental results are calculated first, you can

use the abort feature to terminate unwanted calculations as well

as to escape from command mistakes.  For example, if you are

interested only in obtaining input impedance, SWR, gain, and

F/B, you can press Esc after these results are displayed to

terminate pattern generation.  If you later press G, AO

redisplays results already calculated and resumes pattern

generation where it left off.

If it takes more than one minute to fill and factor the

mutual-impedance matrix, AO beeps when it's done to alert you

that results are ready.

---- AUTOMATIC FREQUENCY SWEEP ---------------------------------

AO can automatically analyze a design at discrete

frequency steps across a frequency range.  It can generate a

plot file at each frequency.  To set this up, use the Frequency

Menu to enter start, stop, and step values for the sweep range.

Then press G for analysis only or P for analysis and plot-file

generation.  When you press P, AO displays the last-frequency

plot after generating all data.  When you press G, AO does not

compute analysis parameters that require pattern generation,

like beamwidth.  This allows very rapid frequency sweep.  AO

records swept-frequency results in the RUN file.

To return to single-frequency mode, set the stop or step

frequency to zero.

27

---- BATCH MODE ------------------------------------------------

Sometimes it's very convenient to run AO unattended from

a DOS batch file.  For example, you might do lengthy analysis of

several complex antenna models overnight.

In batch mode AO runs and terminates automatically

without keyboard intervention.  You can do several AO runs from

one batch file by saving results in separate RUN and plot files.

Here's an example of batch mode:  AO BATCH=G DIPOLE.

Here, AO loads DIPOLE.ANT, runs as if you had pressed G but

aborted the pattern search, and then terminates.  BATCH=G yields

the same results as pressing G during automatic frequency sweep:

AO computes impedance, SWR, losses, efficiency, gain and F/B,

but not beamwidth or maximum-sidelobe level.  Results are saved

in the RUN file.  AO BATCH=P DIPOLE runs as if you had pressed

P:  AO calculates analysis parameters, generates a 2-D plot file

(but does not display it), and then terminates.  AO BATCH DIPOLE

is the same as AO BATCH=P DIPOLE; it's the form you'll probably

use most often.

You can execute more than one command per run, for

example:  AO BATCH=GDN DIPOLE.  Here, AO calculates gain, etc.,

tabulates the far field and near field, and then terminates.

Only the G, P, D, and N commands can be used with BATCH=.

In batch mode the D and N commands use default menu

settings for angle limits, field locations, etc.  To change the

default values, modify the D and N menus before entering batch

mode.  Then use Save Options & Settings to save these values in

AO.INI, perhaps renaming it.  You can recopy saved settings to

AO.INI during a batch run like this:

AO BATCH DIPOLE          Use existing settings

COPY RUN DIPOLE.RUN      Save dipole results

COPY QUAD.INI AO.INI     New settings for quad run

AO BATCH=D QUAD          Tabulate far field for quad

COPY RUN QUAD.RUN        Save quad results

COPY YAGI.INI AO.INI     Settings for Yagi run

AO BATCH=ND YAGI         Compute near and far fields

To perform automatic frequency sweep with BATCH=G or

BATCH=P, specify start, stop, and step frequencies on the

frequency line in the antenna file.  Frequency sweep does not

work for BATCH=D or BATCH=N.

If AO detects an error during batch mode, it records it

in the RUN file and then terminates so that processing can

continue.  It also sets ERRORLEVEL to 1.  You can use the IF

ERRORLEVEL statement in a batch file to take alternate action

whenever an error occurs.  You can abort the current analysis

during batch mode by pressing Esc.

Due to its complex setup, the optimizer cannot be run

from batch mode.

28

---- ALGORITHM LIMITATIONS AND CORRECTIONS ---------------------

The MININEC antenna-analysis algorithm is reasonably

accurate when modeling thin, straight-wire antennas at HF.

However, the algorithm does have inherent biases and limitations

that can affect accuracy in many circumstances.  AO contains

special compensation for several sources of error that are

particularly important.

Frequency Offset

MININEC exhibits an inherent frequency offset for all

models.  This error is small for very thin wires but increases

rapidly with wire diameter.  This effect can bias results at HF,

can cause serious error at VHF, and renders MININEC useless for

practical work at UHF and above.  Gain, pattern, and impedance

characteristics of real antennas occur lower in frequency than

calculated.  Frequency offsets ranging from 0.25% to 2% have

been observed for Yagi models with element diameters from .0001

to .01 wavelength.

AO includes special compensation for this frequency-

offset error.  The correction frequency-calibrates AO to NEC,

the accurate but complex Numerical Electromagnetics Code that

inspired the simpler MININEC algorithm.  Frequency-offset

compensation normally should be enabled.  It can be disabled

from the Options Menu for testing.

With the frequency-offset correction enabled, AO gives

accurate, low-offset results for most Yagis when just 10

segments per halfwave are used.

Bent Wires

Wires joined at an angle cause an internal geometric

distortion in MININEC.  This distortion can result in large

frequency-offset errors unrelated to the frequency-offset effect

described above.  Bent-wire junction effects can cause serious

inaccuracy when modeling loops, cubical quads, rhombics,

groundplanes, etc.  AO contains a special correction for this

problem that works without altering segmentation or increasing

computation time.  The correction can be disabled from the

Options Menu for testing.

With the frequency-offset and bent-wire junction

corrections both enabled, groundplanes and cubical quads with 16

segments/halfwave and equilateral-triangle delta loops with 22

segments/halfwave show little or no frequency offset from NEC

results.  (However, you may need to increase segmentation

density when a source is located at a bent-wire junction.  It's

important to test convergence for unfamiliar geometries.)

29

Dissimilar Segment-Lengths at Wire Junctions

As described earlier, AO can exhibit gross errors when a

short segment is connected to a much longer one.  Connected-

segment lengths normally should differ by no more than a factor

of about 2.  To overcome this limitation, enable automatic

segment-length tapering from the Options Menu.

Ground Losses

AO calculates wire currents assuming perfect-

conductivity earth for antennas modeled over ground.  Horizontal

antennas located less than about 0.2 wavelength above real earth

incur ground losses not calculated by AO.  Because of these

losses, the actual impedance will be higher than calculated and

the realizable gain lower.  A similar situation occurs for

ground-mounted vertical antennas with poor ground systems.

Element-Mounting Effects

The method used to mount elements to a boom can affect

antenna performance, particularly for parasitic arrays at VHF/

UHF.  Conductive mounting plates or through-the-boom mounting

increase element effective diameter and raise resonant

frequency.

To model conductive mounting plates, use the YO Yagi

Optimizer program or refer to "Physical Design of Yagi Antennas"

by Dr. David B. Leeson, W6QHS, published by the ARRL.

To account for through-the-boom mounting of Yagi

elements, use the formula below to shorten measured element

lengths before modeling.  Conversely, when constructing a Yagi,

use the formula to increase calculated element lengths.

For noninsulated through-the-boom mounting, measurements

by Guenter Hoch, DL6WU, have been curve-fitted by Ian White,

G3SEK, to yield the following boom-correction formula for Yagis:

C = 25.195B - 229B^2

C is the correction factor expressed as a fraction of

boom diameter B in wavelengths.  B^2 means B-squared.  For

example, a .01-wavelength diameter boom requires an element-

length correction of 23% of the boom diameter.  The experimental

data underlying this formula were derived from booms with

diameters less than .055 wavelength.  G3SEK indicates that

correction factors for insulated, through-the-boom mounting are

close to 50% of C.

30

Closely Spaced Wires

With some care, you can model antennas with closely

spaced wires like embedded transmission lines.  Higher-than-

normal segmentation density may be required for these models.

Experiments with a terminated, 600-ohm transmission-line

model and with a gamma-match model yielded accurate, converged

results only when segment lengths of the closely spaced wires

were no more than twice the wire spacing.  This segmentation was

required even though automatic segment-length tapering was used

to smoothly transition the segmentation to the rest of the

structure.

On the other hand, Zepp and Lazy-H models with embedded

transmission lines give accurate results even with segments

considerably longer than twice the line spacing.  LPDA models

give reasonable results under the same conditions but converge

better with more segments.

Be skeptical of results for any model with closely

spaced wires until you've carefully checked both algorithm

convergence and wire currents.  Check convergence by increasing

the segmentation density and noting changes in the results.

Look for erratic current magnitude along closely spaced wires.

This is a sure sign that they have too few segments.  You can

set a minimum number of segments for closely spaced wires by

specifying a positive value on the wire lines.  Be sure to

enable automatic segment-length tapering.

---- MEMORY ----------------------------------------------------

The maximum number of pulses for an antenna model is

limited by available memory.  The following terms are used in

this section:

Conventional memory     0 to 640KB

Upper memory            640KB to 1MB

Extended memory         1MB up

High Memory Area        1MB to 1MB+64KB

AO-Amateur

The amateur version of AO uses conventional memory.  A

maximum of 225 pulses is available (450 for free-space symmetric

models).  The actual number available on your particular system

depends on the amount of free conventional memory.  To make more

memory available, you can remove TSRs, device drivers, DOS

shells, or other items loaded by CONFIG.SYS or AUTOEXEC.BAT.

You also can free-up conventional memory by using a memory

manager to load items into upper memory.  AO-Amateur runs at

full speed when used with a memory manager.

31

AO-Professional

The professional version of AO can use either

conventional memory or extended memory.  To access extended

memory, an XMS driver must be installed.  The HIMEM.SYS program

that comes with DOS is such a driver.  XMS allows multiple

programs to share extended memory without conflict.  For

example, a disk cache, a RAM disk, and AO-Pro can all use

extended memory under XMS.  If you've installed HIMEM.SYS to

load DOS high (in the High Memory Area), it's all set for use

with AO-Pro.  See your DOS manual for information on installing

HIMEM.SYS.

When XMS is not available, AO-Pro uses conventional

memory.  AO-Pro also uses conventional memory whenever more of

it is available than extended memory.  This might occur, for

example, when a disk cache has been allocated all of extended

memory.  All caches and RAM disks provide some means to control

the amount of memory they use.  Often you can dynamically reduce

their memory usage from the DOS command line without rebooting.

When running AO-Pro with large models, you may wish to minimize

other uses of extended memory to maximize model capacity.

AO-Pro is not compatible with memory managers that put

the CPU into virtual-8086 mode.  V86 mode prevents AO-Pro from

using protected mode to access extended memory.  Examples of

incompatible memory managers are EMM386, QEMM-386, and 386MAX.

Windows and OS/2 also run DOS programs in V86 mode.  If AO-Pro

detects V86 mode on startup, it displays "Incompatible operating

system or memory manager" and terminates.  You must either

disable your memory manager or install one that does not use V86

mode.

The Last Byte Memory Manager from Key Software Products

can load TSRs, device drivers, and many other items into upper

memory without forcing the CPU into V86 mode.  TLBMM can make

upper memory available for many (but not all) motherboard chip

sets.  TLBMM is totally compatible with AO and imposes no speed

penalty whatsoever.  TLBMM is shareware.  You can verify

compatibility with your computer before licensing the program.

You can download TLBMM from the KSP BBS at (415) 364-9847.

Alternatively, you can obtain the software by sending a blank,

formatted, high-density disk and a self-addressed, stamped, disk

mailer to Key Software Products, 440 Ninth Ave., Menlo Park, CA

94025.  After verifying that it works on your computer, you can

license the software for $29.95.

The following table gives the maximum number of pulses

for AO-Pro as a function of total memory.  These numbers assume

that AO-Pro has access to everything above the High Memory Area.

Double the number of pulses for free-space symmetric models.

32

Memory         Pulses

64 MB           2872

32              2013

16              1399

8               953

4               620

2               350

---- SET COMMANDS ----------------------------------------------

DOS provides a convenient way to specify configuration

information to AO.  The DOS SET command places information you

supply into the DOS environment in memory where it can be

retrieved later by a program.  It's convenient to put SET

commands in your AUTOEXEC.BAT file so they'll be executed

automatically every time the computer boots.  You can enter SET

commands in upper or lower case.  It's best not to leave spaces

around the = in SET commands.

You can see what's in the DOS environment by typing the

following:

SET

You can eliminate a SET parameter P by typing this:

SET P=

If you use lots of SET commands, it's possible to run

out of environment space.  To enlarge it to 1024 bytes, put a

line similar to this one in your CONFIG.SYS file:

SHELL=\COMMAND.COM /E:1024 /P

1.  Text Editor

AO is set up to automatically call the ED program

whenever the E command is given to edit an antenna file.  This

simple, easy-to-use text editor works with small files like

antenna files.  However, you may prefer to use your own editor

or wordprocessor.  To identify a different editor to AO, do the

following:

SET EDITOR=Editorname

The EXE or COM extension is not needed.  You can specify

a drive and path.  When the E command is given, AO appends the

name of the current antenna file to Editorname and calls the

program.  After the file has been edited, AO rereads it.  All

antenna parameters are reset to those in the edited file.  The

RUN file is overwritten.

33

Many editors permit a line number to be specified along

with the filename.  The editor will initialize its cursor at the

specified line.  When AO detects an antenna-file error, it can

pass the offending line number to your editor.  This is very

handy for quick fixes.  AO automatically passes a line number to

ED.  To set this feature up for a different editor, specify the

required string in Editorname.  Put # where your editor expects

the line number.  AO will substitute the actual line number for

# when it calls the editor.  For example, to invoke the Norton

Editor this way, use:

SET EDITOR=ne +#

For BRIEF use:

SET EDITOR=b -m"goto_line #"

If you use a wordprocessor for editing, be sure to

generate a standard DOS text file instead of a file in

wordprocessing format.  Files in wordprocessing format contain

embedded control codes that will confuse AO.

2. Subdirectories

Once you accumulate many antenna and plot files, it's

nice to organize them into subdirectories.  You might use the

current directory for antenna experiments, saving optimized

antenna files and plots elsewhere.  You can specify

subdirectories to AO with SET commands.  The subdirectories are

used only for reading files; AO always writes the RUN file,

OUT.ANT (from the optimizer), and plot files to the current

directory.

Define subdirectories like this:

SET ANT=BEAMS LOOPS VHF .

SET PLT=. HFPLOTS VHFPLOTS BEAMS

The dot represents the current directory.  It can appear

anywhere (or nowhere) in the list.  You can define as many

subdirectories as you like.  Often it's convenient to use the

same set of subdirectories for both antenna and plot files.  You

can organize subdirectories by antenna type, frequency range,

designer, etc.

Without a SET command, each subdirectory list defaults

to just the current directory.

When you start AO, it begins by listing files in the

first subdirectory on the list.  The remaining subdirectory

names are listed along with the files.  You can select one to

list its files.

34

You can specify a file from one of the subdirectories on

the AO command line without typing the path.  AO searches the

subdirectories in the order of the subdirectory list.

You can specify any subdirectory (not necessarily from

the list) on the command line and AO will list its antenna

files.  Use AO . to force AO to list files in the current

directory when it's not first on the subdirectory list.

3. Reference dB

To display gain figures in dBd rather than dBi, do the

following:

SET DB=dBd

To use dBd for free-space models and dBi over ground, do

this:

SET DB=dBdi

Gain in dBd is referenced to the peak gain of a halfwave

dipole in free space.  Gain in dBi is referenced to an isotropic

radiator in free space.  An isotropic antenna radiates equally

in all directions.  A dipole has 2.15 dB gain over an isotropic

antenna, so AO converts from one gain reference to the other by

adding or subtracting 2.15 dB.

4.  Printer Type

AO defaults printed output for HP LaserJet or DeskJet

printers in portrait mode.  To try a landscape plot (not

available on all printers), use:

SET PINS=HPLJ L

For a bigger landscape plot, use:

SET PINS=HPLJ LB

AO normally draws LaserJet/DeskJet plots with a border.

To eliminate the border, add an X to other options, for example:

SET PINS=HPLJ LBX

AO can transfer compressed data to PCL 5 laser printers

for faster output.  To try this, add a C to other options, for

example:

SET PINS=HPLJ LBXC

For a 9-pin dot-matrix printer, do the following:

SET PINS=9

35

Some 9-pin printers don't recognize the special line

spacing command AO uses to print an exact screen image.  If your

printer won't print AO screens, try the following:

SET PINS=9ALT

Even with 9ALT, AO may be unable to print to certain old

9-pin printers.

For a 24-pin dot-matrix printer, use:

SET PINS=24

5.  Printer Port

Graphics screens print on LPT1 by default.  To print

graphics on another port, use one of the following:

SET LPT=LPT2

SET LPT=LPT3

6.  .PCX Output

By default, AO generates .PCX screen images with 640 by

480 pixels.  These images can be directly incorporated in

documents.  However, the images won't be centered on fax pages

because they have no margins.  You can add margins with the

following command:

SET PCX=left top bottom

The three numbers specify margin size in inches.  The

bottom margin is optional.  You can use it to pad the image to

form a complete page if your fax software doesn't do this

automatically.  Try left = .8 and top = 2.35 for 8.5" x 11"

pages.

You can create a double-size .PCX image by adding the

keyword BIG to the line (upper or lower case).  Use BIG for

high-resolution fax mode.

7.  Automatic Segment-Length Tapering Ratio

To change the length ratio for automatic segment-length

tapering from the default value of 2, specify a decimal number

as follows:

SET RATIO=Ratio

8.  Critical-Command Confirmation

Normally AO asks for confirmation to load another

36

antenna file, to edit a file, or to quit only after analysis has

taken longer than 5 minutes to complete.  This yields quick

command response for small models while providing protection

against accidental destruction of time-consuming results.  You

can change the 5-minute default by specifying a decimal number

of minutes as follows:

SET CONFIRM=Time

9.  Linear-dB Polar Plots

AO normally displays polar patterns using a log-dB

scale.  If you prefer the linear-dB scale, you can make it the

default with:

SET POLAR=Linear

10.  Display Typeface

You can use the standard serif typeface built into your

VGA card with the following:

SET TYPEFACE=Serif

11.  VGA Compatibility

AO can use a dual-paging scheme to provide smoother

graphics animation by fully buffering all screen changes.  This

scheme requires a compatible VGA card with 512K video memory.

If your card is incompatible or has less memory, dual paging

will cause extraneous graphics images.  Dual paging eliminates

flicker during geometry-screen rotation and the blink when 2-D

plot screens change.  To try dual paging, do the following:

SET PAGES=2

AO programs the VGA overscan register to extend the

background color into the overscan region surrounding the active

screen.  This improves the appearance of text at screen edges.

However, some VGA cards generate the wrong overscan color,

yielding a distinct, off-color border.  To fix this problem, try

the following:

SET OVERSCAN=FIX

12.  Screen Bounce

When switched from graphics to text mode, the screens of

many monitors bounce or break up during resynchronization.  AO

can blank the screen for a short period while the monitor

37

settles.  To try this, specify a short delay in seconds (like

0.1) as follows:

SET BOUNCE=Delay

13.  Default Ground Constants

To change the default single-zone ground constants, do

the following:

SET GND=D C

D is dielectric constant and C is conductivity in mS/m.

D = 13 and C = 5 when GND is not set.  For a perfect-

conductivity default, use:

SET GND=0

14.  Far-Field, Low-Level Threshold

To change the level below which relative far-field

values are tabulated as -infinity dB, enter a negative dB value

as follows:

SET THOLD=Level

15.  Typematic Control

When you press and hold a key, the key repeats after a

short delay.  Default PC values are 500 ms delay and a repeat

rate of 10 characters per second (10.9 cps for PS/2).  When

scrolling the RUN file or rotating 3-D graphics, many users

prefer a shorter delay and a faster repeat rate.  AO sets the

delay to 250 ms and the repeat rate to 24 cps.  You can change

these values as follows:

SET TYPEMATIC=delay rate exitdelay exitrate

Delay codes are 0-3 as follows:

0    250 ms     2   750 ms

1    500        3  1000

Rate codes are 0-13 as follows:

0   30.0 cps    7   16.0 cps

1   26.7        8   15.0

2   24.0        9   13.3

3   21.8       10   12.0

4   20.0       11   10.9

5   18.5       12   10.0

6   17.1       13    9.2

38

The exitdelay and exitrate parameters are optional.

When present, AO sets these typematic values on exit to DOS.

When not present, the values on exit are those in effect while

running AO.

To keep AO from modifying your system's typematic

values, do this:

SET TYPEMATIC=0

---- ADDITIONAL INFORMATION ------------------------------------

NOSC Technical Document 938 contains a detailed

description of the original MININEC program, including

mathematical formulation of the analysis algorithm, model

validation against empirical data, error analysis, and BASIC

program listing.  This highly technical document is intended for

readers with a background in advanced mathematics and

theoretical electromagnetics.  The 100+ page manual is available

from the U.S. Dept. of Commerce for $22.95.  It can be ordered

by mail with a check or by phone with a credit card.  Request

NTIS document number ADA181682 from:

U.S. Dept. of Commerce

National Technical Information Service

5285 Port Royal Rd.

Springfield, VA 22161

(703) 487-4650

39

INDEX

#  3

+ - * / operators  17

-120 dB  21

.PCX file  27

.PLT  23

2-D Pattern Generation  23

2-D Pattern Plots  23

225 pulses  31

3-D Geometry/Current Display  21

3-D Pattern Display  22

386MAX  32

6061-T6  14

6063-T832  14

A command  2

A key  24

Abbreviations  3

Aborting Calculations  27

Absolute far field  21

Accuracy  4, 5, 29

Additional Information  39

Algorithm Limitations  29

Align antennas  23

Aluminum  14

Amateur version  31

Ambiguity  22

American Wire Gauge  3

Analysis over ground  10

Annealed bare copper wire  3

Annealed copper  14

Annotate  9, 11, 26

ANT  2

Antenna file  1

Antenna Files  2

Antenna image  10

AO BATCH  28

AO PLOT  1, 23

AO.INI  26

Array element  12

ARRL  30

ARRL log-dB scale  24

Arrow keys  1

Aspect ratio  25

Assembly language  1

Assumed to be zero  9

AUTOEXEC.BAT  31, 33

Automatic frequency sweep  15, 27, 28

Automatic Segment-Length Tapering  5, 31

Automatic Segment-Length Tapering Ratio  36

Automatic segmentation  5

Automatic Wire Segmentation  5

Azimuth-angle convention  3

40

Barber's thinning shears  22

Batch file  28

Batch Mode  28

Beamwidth  23

Beep  27

Bent wire  3, 29

Bias  29

Bias results  29

BIG  36

Blank lines  3

Blink  37

Blockage of radiation  11

Boom  30

Boom-correction formula  30

Brass  14

BRIEF  34

British spellings  3

Bypass the Main Menu  2

C  8

C = 25.195B - 229B^2  30

C key  25

Calculated gain  11

Calculated impedance  11

Calibrations  24

Cartesian coordinate system  3

Caution  11

Center  6

Center pulse  6

Chemically treated  11

Circular  25

Circularity of a spiral  21

Closely Spaced Wires  31

Clutter  25

Code optimization  1

Coefficients  9

Coincident endpoints  4

Colors  26

COM  33

Comma  3, 6

Command key  1

Command line  1, 23, 32, 35

COMMAND.COM  33

Comment lines  3

Compare command  20

Compare mode  24

Comparing Plots  25

Compass bearings  3

Complete record of the analysis session  2

Complex conjugate  15

Computational noise  21

Conductive mounting plates  30

Conductivity  11, 12, 14, 38

Conductor material  14

CONFIG.SYS  31, 33

Connected segments  5

41

Connected-segment lengths  30

Connections  4

Conventional memory  31

Convergence  4, 5, 29, 31

Coordinate System  3

Copper  14

Correct patterns  23

Correction factor  30

COS  17

Cosine  17

Critical-Command Confirmation  36

Current  4, 10, 11, 31

Current directory  1, 23, 26, 34

Current distribution  21

Current phase  22

Current sources  7

Current trace  21

Current-return path  7

Currents section  7

D and N commands  2

DBd  12, 35

DBdi  35

DBi  12, 35

Default  6, 7, 23, 36

Default ground characteristics  11, 12

Default Ground Constants  38

Default Menu Settings  26, 28

Delta loops  29

Dense segmentation  5

Descriptive phrase  9

DeskJet  26

Device drivers  31, 32

Dielectric constant  11, 12, 38

Dielectrics  1

Diffraction  11

Dimension overrides  3, 17, 18

Dimensions  3, 16

Dipole  12, 35

Directory  1

Disk cache  32

Display Typeface  37

Dissimilar Segment-Lengths  30

Distance from the antenna  21

DL6WU  30

DOS  32

DOS environment  33

DOS shells  31

Dot  24, 34

Dot-matrix printers  26

Dual-paging scheme  37

Dynamic range  26

E command  2, 33

Earth  11, 30

ED program  33

42

Edit  2, 37

Editor  33

Effective diameter  30

Efficiency  14

Eject  26

Electric and magnetic fields  20

Electromagnetic field  19

Electromagnetic-field component  26

Element-length correction  30

Element-Mounting Effects  30

Elevation angle  23

Elevation variation  11

Elliptical  25

Ellipticity  20

Embedded transmission lines  31

EMI  20

EMM386  32

End  1

End1  6, 22

End2  6, 22

Endpoints  3

Enter  1

Environment  12

Environment space  33

Epson-compatible  26

Erratic current  31

Error  7, 10, 16, 21, 28, 29, 34, 39

ERRORLEVEL  28

Esc  1, 26, 27

EXE  33

Experiment  26

Expressions  17

Extended-ASCII characters  27

Extraneous graphics images  37

F  8

F5  26

F9  27

Far Field  21

Farads  9

Fax  27

Feedline  7, 15

Feedpoint  7

Ferrites  1

Ferromagnetic material  14

Field components  20

Filename  1

First wire of a junction  8

Flicker  37

Formula  30

Free conventional memory  31

Free space  10

Free wire end  7

Free-form  3

Free-space symmetric  31

Free-Space Symmetric Antennas  10

43

Frequency  4, 8, 9, 15, 28, 30

Frequency Offset  29

Frequency steps  27

Frequency sweep  28

G3SEK  30

Gain figures  12

Gamma-match model  31

Generating Plot Data  25

Geometric distortion  29

Graphics animation  37

Graphics Image Files  27

Graphics screens  36

Ground  4, 11

Ground Losses  30

Ground screen  11

Ground zone  11, 12

Ground-current losses  11

Ground-reflection factor  11

Ground-reflection wave  12, 21

Grounded wire end  4, 7

Groundplane  11, 13, 29

H  8, 20

Half-length pulse  4

Half-segment  4

Half-wavelength of wire  5

Halfwave dipole  35

Halfwave of wire  4

Height  3, 11, 13

Help  24

Henries  9

HF  29

HF-antenna performance  19

Hide the Main Menu  2

Highlighted file  1

Hilltops  11

HIMEM.SYS  32

Hoch  30

Home  1

Horizon  3, 12

Horizontal antennas  11, 30

Horizontal Component  20

Horizontal plane  3

HP LaserJet  26

IACS  14

Ignore wires  4

Image wires  10

Impedance  11, 30

Impedance loads  9

In-phase current  10

Inaccurate results  5

Incompatible operating system or memory manager  32

Infinity  11, 13, 21, 24, 38

Input impedance  15

44

Input power  21

Input resistance  8

Interference-level analysis  21

International Annealed Copper Standard  14

Interpolation  23

Isotropic  12, 35

Junction  4, 6, 7, 29

Key repeats  38

Key Software Products  32

Keyword  3, 6, 9

L  8

Laplace Transform loads  9

Lazy-H  31

LC  20

Leeson  30

Left-Circular Component  20

Length of each segment  4

Lengthy results  2

Level  38

Lightbar  1

Limit  4, 8, 9, 31

Limitations  29

Line number  34

Linear-dB scale  24, 37

List plot files  23

Load another antenna file  36

Load coincident with a source  9

Load losses  14

Load syntax  8

Loads  6, 8, 10

Lobe maxima  12

Log-dB scale  24, 37

Logan  1

Loops  29

Losses  11, 14

Lossy capacitor  8

Lossy inductor  8

Lossy trap  8

LPDA  31

LPT  36

Lumped loads  8

M key  21

Magnitude  21

Magnitude-only currents  21

Major  20

Major-Axis Component  20

Manually segmenting driven wires  7

Mathematical formulation  39

Maximum number of pulses  4, 31, 32

Maximum sidelobe level  23

Memory  4, 31

Memory manager  31

45

Menu settings  26

Midpoints  4

MININEC 3.13  1

Minor  20

Minor-Axis Component  20

Mirror-image  10, 23

Mirrors the antenna  10

Model validation  39

Motherboard chip sets  32

Mounting plates  30

MS/m  12

Multiple sources  8

Multiwire junction  7

Mutual-impedance matrix  7, 20

National Technical Information Service  39

Near Field  20

Near field over ground  20

NEC  29

Negative gain values  12

Negative input resistance  8

Networks  9

NH  8

Norton Editor  34

NOSC Technical Document  39

Number of sources  8

Number of wires  4

Number-of-loads line  6

Number-of-sources line  7

Number-of-wires line  14

Numerical Electromagnetics Code  29

O key  25

Oblique wires  19

Off-center feedpoint  7

Ohm-m  14

Ohms  8, 9

On-axis response  23

Optimizing a design  12

Optimizing an antenna pattern  23

Optimizing capabilities  1

Option  23, 24

Options Menu  5, 6, 15, 23

Order  4, 8

Origin  11

OS/2  32

Other  1

Out-of-phase current  10

Overlap  4

Overlay  20, 24, 25

Overwritten  2

P command  25

Path  33

Pattern generation  21

Patterns over ground  25

46

PCX file  27

PCX Output  36

Perfect-conductivity ground  12, 20

Permeability  14

PF  8

PgDn  1

PgUp  1

Phased arrays  7

Phasors  21

Phosphor Bronze  14

Physical Design of Yagi Antennas  30

Plot file  1, 25

PLT  25

Plural  3

Points in space  3

POLAR environment variable  24

Polar plot  24, 37

Polar Plots  24

Polarization Component  19

Polarization tilt  20

Polarization-Ellipse Circularity  20

Polynomial  9

Poor ground systems  11, 30

Preferred settings  26

Preserves plot files for later comparison  25

Print  26

Print graphics  36

Printer Port  36

Printer Type  35

Printing The Screen  26

Problem junctions  5

Professional version  32

Protected mode  32

PrtSc  2, 26

Pulse  4, 10

Pulse allocation  10

Pulse Identification  6

Pulse number  6, 10

Pulses  10

Q  8

QEMM-386  32

Quads  29

Quicker results  5

Quit  37

R  8

R command  2

R key  25

Radial lines  24

Radial scales  24

Radial wires  11

Radius  11, 13

RAM disk  32

Ratio  36

RC  20

47

Reactance  9

Rear lobe  23

Recalculation  7

Rectangular Plots  25

Reference antenna  12

Reference dB  35

Relative far field  21

Rename  2

Reorder  4

Reorders wires  8

Resistance  9

Resistive load  11

Resistivity  14

Resistor  8

Resolution  22, 23, 26

Review plots  23

RF generator  7

RF-hazard analysis  20

Rhombics  29

Right-Circular Component  20

RLC loads  8

RLC syntax  8

Rockway  1

Rotate an object  17

Rotation angle  21

Run AO unattended  28

RUN file  2, 7, 10, 33, 34

Running AO  1

S command  26

Salt water  12

Save Options & Settings  26, 28

Scientific notation  17

Screen Bounce  37

Screen Colors  26

Search-and-replace  16

Segment junctions  4, 6

Segment lengths  31

Segmentation  5, 8

Segmentation density  31

Segments  4, 29

Segments per halfwave  5

Serif typeface  37

SET ANT  34

SET BOUNCE  38

SET CONFIRM  37

SET DB  35

SET EDITOR  33

SET GND  12, 38

SET LPT  36

SET OVERSCAN  37

SET PAGES  37

SET PLT  34

SET POLAR  37

SET RATIO  36

SET THOLD  38

48

SET TYPEFACE  37

SET TYPEMATIC  38

Settings  26

SHELL  33

Shift an object  18

Shift and Rotate  17

Shift-end command  18

Short wire  5

Sidelobe  23, 24

Signal-coverage studies  21

Silver  14

Simulate ground-current losses  12

SIN  17

Sine  17

Single-frequency mode  27

Single-source models  7

Single-wire models  10

Singular  3

Singular or plural  13

Slice the pattern  22

Sloping ground  11

Soil  11, 13

Source magnitude and phase  7

Sources  6, 7, 10, 15

Sources command  7

Space wave  21

Spaces  3

Spaces, commas, or tabs  9

Sparse segmentation  5

Special cases  5

Speed  32

Speed computation  10

Speed increase  1

Speed pattern-generation  23

SQR  17

Square root  17

Standing-Wave Ratio  15

Steel  14

Step  20

Subdirectories  34

Subdivide  4, 6

Submenu  1

Summary printout  2

Surface wave  21

Surfaces  1

Sweep range  27

SWR  15

SWR reference impedance  15

Symbol definitions  17

Symbol set  27

Symbolic Dimensions  15

Symbolic Expressions  17

Symbols  16

Symmetrical about the X-axis  23

Symmetrical antennas  10

Syntax  9

49

Tabs  3

Telescoping tubing  4

Terminate data entry  1

Terrain  11

Text Editor  33

The Last Byte Memory Manager  32

Thin a pattern  22

Thin wires  29

Threshold  21, 38

Through-the-boom mounting  30

Tilt  20

Time-saving feature  23

TLBMM  32

Too short  4

Total  20

Total Field  20, 26

Trace  21

Transform order  9

Transformer  15

Transmission lines  5, 31

Traps  8

Traveling-wave antennas  8

Traveling-wave currents  21

Trick  6

TSRs  31, 32

TVI  20

Typematic Control  38

U.S. Dept. of Commerce  39

U.S. Naval Ocean Systems Center  1

UH  8

UHF  29

Unconnected wire end  7

Unfamiliar geometries  29

Unidirectional antennas  3

Unit of length  3

Units  8, 9

Upper case  1

Upper memory  31, 32

Upper or lower case  3, 6, 8, 33

Upper-case P  20, 25

V  20

V86 mode  32

Valid results  4

Vertical antennas  12, 30

Vertical Component  20

VGA Compatibility  37

VHF  29

VHF/UHF  30

Video memory  37

View the whole screen  2

Viewing commands  21

Viewpoints  22

Voltage  7

50

Volts/meter  21

Volume resistivity  14

W key  21

W6QHS  30

Warning message  4, 6

Wave polarization  19

White  30

Windows  32

Wire axis  21

Wire currents  21

Wire diameter  3, 29

Wire ends  4, 22

Wire gauge  3

Wire is always straight  3

Wire junction  4, 8

Wire line  6

Wire list  3

Wire losses  14

Wire segmentation  5

Wire spacing  31

Wire-Conductivity Losses  14

Wire-material phrase  14

Wires and Pulses  10

Wires and Pulses section  4, 6, 7

Wires that cross  4

Wires, Segments, and Pulses  3

Wordprocessor  33

X  3

X key  25

X-axis  23

X-axis cutoff  25

X-axis pattern symmetry  22

XMS driver  32

Y  3

Y key  25

Y-axis cutoff  25

YO Yagi Optimizer  30

Z  3

Zepp  31

Zone  11, 13, 38

51