GUY 1.0 Guy-Wire Modeler

The GUY.EXE utility makes it easy to investigate the

effect of guy wires on antenna systems with the AO Antenna

Optimizer or NEC/Wires.

GUY lets you specify guy-wire geometry in simple terms

like attachment height, anchor distance, insulated-section

length, etc.  It then generates a file containing AO wire lines

for all guy-wire sections.  You can maintain guy-wire files for

several tower installations.  GUY can combine a guy-wire file

with an AO antenna file to form a complete system model.  You

can compare results for the composite model with those for the

antenna alone to study guy-wire/antenna interaction.

GUY does not handle AO antenna files that use symbolic

expressions or shifts and rotates.

Preliminaries

GUY works with filenames supplied on the command line.

If one filename is given, GUY creates a guy-wire file with the

specified name plus the extension GUY.  If three filenames are

given, GUY combines the first file (a guy-wire file) with the

second (an antenna file) to generate the third (a composite

antenna file).  Extensions GUY and ANT are not required.  GUY

always asks permission before overwriting an existing file.

GUY normally uses U.S. units.  To use metric units, add

/m to the command line.  If you always use metric units, you can

save keystrokes by running GUY from the following batch file:

GUY %1 %2 %3 /m

You can abort from GUY at any time by pressing Esc.  An

output file is neither created nor overwritten when you abort.

If you forget how to use GUY, type GUY without

parameters to display a quick reference.

Generating Guy-Wire Files

GUY asks for a description of the guy-wire file.  The

description helps identify the file later when you combine it

with an antenna file.  Press Enter for a default description.

Dimensions refer to the centerline of the tower.  You

can refer dimensions to the tower faces without serious error.

GUY generates straight, collinear, unconnected wires to

represent insulated guy-wire sections.  GUY assumes compression

insulators are used which effectively overlap wire ends by one

inch.  GUY lengthens each end an additional half inch to account

for the capacitive-loading effect of the wire loops.

GUY models guy-wire sections with a physically

coincident overlap of two inches even though the wires are

electrically insulated.  This takes advantage of the MININEC

convention that wires are considered to be connected only when

they have common endpoints.  The overlap does not affect model

accuracy; it's just a handy way to account for the physical

properties of compression insulators.

GUY assumes that each guy level has three guy wires

spaced 120 degrees in azimuth.  Each guy wire can have up to 50

insulated sections.  The final guy sections are generated with

their lower ends at ground level.  Long grounded sections may

develop considerable current when the guys are used to support a

vertical radiator.

GUY models the first guy-wire sections attached to the

tower.  The wires are electrically connected at the tower

centerline but the tower itself is not modeled.  Thus, tower

resonances involving the first guy-wire sections are not

modeled.  Torque bars and tower cross-section are not modeled

either, so the effective guy-wire diameter in this region is

much larger than that modeled.  Since the first guy-wire

sections often are close to an antenna, they may critically

affect it.  You should keep these modeling limitations in mind

when the first guy-wire sections are more than a few feet long.

GUY does not model guy-wire conductivity or sag.  These

effects are electrically negligible.

Combining Files

GUY can combine any guy-wire file with any antenna file.

When the antenna file is in free space, the composite output

file is in free space.  However, free space symmetric models are

never generated.  In general, guy wires destroy model symmetry.

GUY asks for a description of the composite output file.

The description becomes the output file title.  Press Enter for

a default description.

Guy-wire files have one set of guys in the +Y direction.

These wires run somewhat parallel to elements of Yagis and

cubical quads when the antenna is aimed in the +X direction.

This orientation usually maximizes coupling between the guy

wires and the antenna.  You may rotate the guy wires with

respect to the antenna to simulate antenna rotation.  This will

reveal additional coupling modes.

Unless you rotate the guy wires exactly 30 degrees, the

composite model will not be symmetrical in azimuth.  In general,

you must disable X-axis pattern symmetry in AO to generate

correct patterns.  Even when rotated 30 degrees, free-space

elevation patterns will not be correct unless symmetry is

disabled.

GUY lets you adjust antenna height in the composite

output file.  This can greatly alter coupling to the guy wires.

GUY detects the presence of two planar antennas and prompts for

the height of each.  For nonplanar antennas like cubical quads,

antenna height is defined as the average of the highest and

lowest wire ends.  This number is boom height for quads, but

this is not true for antennas like delta loops.  In any case,

pressing Enter to accept the default height leaves all

z-coordinate values unchanged.

It's impractical to model all guy-wire sections for most

towers because too many pulses are required.  It's also

unnecessary since remote sections usually have very little

effect.  GUY includes in the composite output file only guy-wire

sections closer to the antenna than a specified distance.

Distance is measured between each end of every antenna wire and

each end of every guy-wire section.

You may specify the number of modeling segments per

half-wavelength of guy wire.  It's not necessary to use the

segmentation density normally used for antenna conductors.

Since guy-wire sections are not likely to be near resonance, you

can get away with fewer segments.

The composite output file uses symbolic dimensions for

antenna height and for guy-wire segmentation.  This lets you

easily experiment with these values from within AO.  Guy wires

use a negative number of segments so that you can employ

automatic segmentation for the antenna conductors without

affecting guy-wire segmentation.